Supplementary data for

Microfabricated Atomic Vapor Cells with Multi-Optical Channels Based on an Innovative Inner-Sidewall Molding Process

Mingzhi Yu a,b, Yao Chen a,b,c,*, Yongliang Wang d, Xiangguang Han a,b,c, Guoxi Luo a,b,c, Libo Zhao a,b,c,*, Yanbin Wang a,b, Yintao Ma a,b, Shun Lu a,b, Ping Yang a,b,c, Qijing Lin a,b,c, Kaifei Wang e,*, Zhuangde Jiang a,b,c

a International Joint Laboratory for Micro/Nano Manufacturing and Measurement Technologies,Xi'an Jiaotong University, Xi'an 710049, China

b School of Mechanical Engineering, Xi’an Jiaotong University, Xi’an 710049, China

c Shandong Laboratory of Yantai Advanced Materials and Green Manufacturing, Yantai 265503, China

d Moon Environment Technology Co., Ltd., Yantai 264000, China

e Emergency Department, The First Affiliated Hospital of Xi’an Jiaotong University, Xi’an 710061, China

* Corresponding authors.

E-mail addresses: yaochen@xjtu.edu.cn (Y. Chen), libozhao@xjtu.edu.cn (L. Zhao), kaifeiw@xjtufh.edu.cn (K. Wang)

FID signal fft and triple Lorentz fitting procedure

clc;

clear all;

a=importdata('E:\data.txt');

a1=a(:,1)

a2=a(:,2)

plot(a1,a2)

%fft

time=a1;

time=time-time(1);

data=a2;

Fs = 1/(time(2)-time(1))-0.1  % Sampling frequency

L = length(data);

T = 1/Fs;                     % Sample time

t = (0:L-1)*T;

figure(1)

plot(t,data)

Y=fft(data)

p2=abs(Y/L);

p1=p2(1:L/2+1);

p3=2*p1(2:120);

f=(Fs*(0:(L/2))/L);  % Frequency resolution：Fs/L

f=f(1:120-1);

figure(2)

plot(f,p3)

cftool

%Triple Lorentz fit parameters of Fig.13(b)

A1 =    0.009521; % Fitting coefficient

A129 =    0.008788; % Fitting coefficient

A2 =     0.04008; % Fitting coefficient

A3 =    -0.06402; % Fitting coefficient

T1 =      0.1704; % Resonance peak spectral line width of 131Xe-1

T129 =      0.3033; % Resonance peak spectral line width of 129Xe

T2 =       2.841; % Resonance peak spectral line width of 131Xe-2

T3 =       5.009; % Resonance peak spectral line width of 131Xe-3

b =    0.002167; % Fitting coefficient

deltaf =     0.05224; % Electric quadrupole split

v129 =       6.038; % precession frequencies of 129Xe

v131 =       1.734; % precession frequencies of 131Xe

aaa=zeros(1,length(f));

for i=1:length(f)

v=f(i);

aaa(i)=A129*T129/(2*pi)/((v-v129)^2+(T129/2)^2)+A1*T1/(2*pi)/((v-v131)^2+(T1/2)^2)+A2*T2/(2*pi)/((v-v131-deltaf)^2+(T2/2)^2)+A3*T3/(2*pi)/((v-v131+deltaf)^2+(T3/2)^2)+b;

end

figure(3)

plot(f,aaa)

%Triple Lorentz fit parameters of Fig.13(d)

A1 =    0.008241;

A129 =    0.005224;

A2 =   0.0006596;

A3 =    0.002835;

T1 =      0.1669;

T129 =         0.3;

T2 =      0.4546;

T3 =       1.233;

b =    0.000529;

deltaf =      0.4331;

v129 =       6.048;

v131 =       1.739;

aaa=zeros(1,length(f));

for i=1:length(f)

v=f(i);

aaa(i)=A129*T129/(2*pi)/((v-v129)^2+(T129/2)^2)+A1*T1/(2*pi)/((v-v131)^2+(T1/2)^2)+A2*T2/(2*pi)/((v-v131-deltaf)^2+(T2/2)^2)+A3*T3/(2*pi)/((v-v131+deltaf)^2+(T3/2)^2)+b;

end

figure(3)

plot(f,aaa)