Research

Frontier Research on Carbon Neutrality—Article

Investigation into the Methodology and Implementation of Life Cycle Engineering under China's Carbon Reduction Target in the Process Industry

Mingyang Li, Feng Gao, Zuoren Nie*, Boxue Sun, Yu Liu, Xianzheng Gong

National Engineering Laboratory for Industrial Big-data Application Technology, College of Material Science and Engineering, Beijing University of Technology, Beijing 100124, China

* Corresponding author.

E-mail address: zrnie@bjut.edu.cn (Z. Nie).

# 1. Advanced technologies of carbon reduction

# Table S1

# The advanced technology applied in the primary aluminum production [1].

| Number | Process | Advanced Technology | Potential |
| --- | --- | --- | --- |
| T1 | Alumina production | Low A/S clinker sintering | Energy consumption reduction of 628MJ/t-Al2O3 |
| T2 |  | System energy saving optimization | Energy consumption reduction of 1826MJ/t-Al2O3 |
| T3 |  | Equipment upsizing | Energy consumption reduction of 511MJ/t-Al2O3 |
| T4 |  | Calcium-free solubilization technology for monohydrate sclerite bauxite ores | Energy consumption reduction of 0.3GJ/t-Al2O3 |
| T5 |  | Energy saving technology of alumina evaporation process system | Energy consumption reduction of about 0.3GJ/t-Al2O3 |
| T6 |  | Carbon and alkali reduction technology in alumina production system | Energy consumption reduction of 0.15GJ/t-Al2O3 |
| T7 |  | Technology of improving alumina product quality by systematic regulation | Reduce carbon emission by 75kgCO2/t-Al |
| T8 | Carbon anode production | Energy saving and carbon reduction technology of carbon anode thermal furnace for aluminum | Reduction of natural gas consumption by 10Nm3/t-C for ton of anode roasting |
| T9 |  | Energy saving and carbon reduction technology of new structure carbon kiln - new fire passage structure | Natural gas consumption reduced by 20 Nm3//t-C on average |
|  |  | Energy saving and carbon reduction technology of new structure carbon kiln - vertical anode continuous roasting technology |  |
|  |  | Energy saving and carbon reduction technology of new structure carbon kiln - horizontal anode continuous roasting |  |
| T10 | Electrolysis | High current electrolysis technology -12000 | Electrolysis power consumption can be reduced to 12000 kWh |
| T11 |  | Technology of carbon anode for aluminum production, quality improvement and carbon reduction | Reduce the anode consumption of electrolytic aluminum production by about 10kg/t-Al |
| T12 |  | "Holographic" operation control technology | Electricity consumption is reduced by 250 kWh |

# 2. Performance of different aluminum alloy

Table S2 is the performance of primary aluminum alloy (PAA) and recycled aluminum alloy (RAA) [2].

Table S2. Mechanical properties of two aluminum alloys

| Alloy | Tensile strength (MPa) | Yield strength (MPa) | Elongation (%) |
| --- | --- | --- | --- |
| PAA | 256 | 204 | 4.3 |
| RAA | 254 | 202 | 4.2 |

# 3. Different indicators result of three aluminum alloy

The result of different indicators of three aluminium alloy was summary in Table S3.

Table S3. The results of each index of the three aluminum alloys

| Category | RCI (GJ/t) | CEI (t/t) | Performance |
| --- | --- | --- | --- |
| PA | 366.49 | 17.4 | 1 |
| PATA | 320.25 | 14.8 | 1 |
| RA | 48.44 | 2.15 | 0.986 |

# 4. Process and results of comprehensive characterization matrix model

The four group of weight coefficient selected were 1/3, 1/3, 1/3; 0.3, 0.3, 0.4; 0.3, 0.4, 0.3; 0.4, 0.3, 0.3. (R, C, P)

The calculate process and results was shown as follow.

A1=1/31/31/3111  0.870.851 0.130.121.014=1, 0.907,0.421

A2=0.30.30.4111  0.870.851 0.130.121.014=(1, 0.916, 0.4806)

A3=0.30.40.3111  0.870.851 0.130.121.014=(1, 0.901, 0.3912)

A4=0.40.30.3111  0.870.851 0.130.121.014=(1, 0.903,0.3922)

# 5. Program code

import csv

PA = {'name': 'PA_value', 'RCI': 1, 'CEI': 1, 'PER': 1}

PATA = {'name': 'PATA_value', 'RCI': 0.87, 'CEI': 0.85, 'PER': 1}

RA = {'name': 'RA_value', 'RCI': 0.13, 'CEI': 0.12, 'PER': 1.014}

file_pa = open("pa.csv", mode='w', newline='')

write_pa = csv.writer(file_pa)

file_pata = open("pata.csv", mode='w', newline='')

write_pata = csv.writer(file_pata)

file_ra = open("ra.csv", mode='w', newline='')

write_ra = csv.writer(file_ra)

line_ra_count = 0

line_pa_count = 0

line_pata_count = 0

for a in range(0, 101):

for b in range(0, 101 - a):

c = 100 - a - b

value_R1 = PA['RCI'] * a

value_E1 = PA['CEI'] * b

value_P1 = PA['PER'] * c

value1 = value_P1 + value_E1 + value_R1

value_R2 = PATA['RCI'] * a

value_E2 = PATA['CEI'] * b

value_P2 = PATA['PER'] * c

value2 = value_P2 + value_E2 + value_R2

value_R3 = RA['RCI'] * a

value_E3 = RA['CEI'] * b

value_P3 = RA['PER'] * c

value3 = value_P3 + value_E3 + value_R3

if value1 < value2 and value1 < value3 or value1 == value2 and value1 < value3 or value1 == value3 and value1< value2:

listpa = [a, b, c, value_R1, value_E1, value_P1, value1]

write_pa.writerow(listpa)

line_pa_count += 1

if value2 < value1 and value2 < value3 or value1 == value2 and value2 < value3 or value2 ==value3 and value2 < value1:

listpata = [a, b, c, value_R2, value_E2, value_P2, value2]

write_pata.writerow(listpata)

line_pata_count += 1

if value3 < value2 and value3 < value1 or value1 == value3 and value3 < value2 or value3 == value2 and value3 < value1:

listra = [a, b, c, value_R3, value_E3, value_P3, value3]

write_ra.writerow(listra)

line_ra_count += 1

file_pa.close()

file_ra.close()

file_pata.close()

print(f"i(PA)={line_pa_count}，i(PATA)={line_pata_count}，i(RA)={line_ra_count}")

# References

# [1] Li M, Gao F, Sun B, Liu Y, Gong X, Nie Z. Zero carbon-emission technology route construction and multifactor analysis of aluminum production in China. J Clean Prod 2022;370:133535.

# [2] Ge S, Liu Y, Ye Z, Lu Y, Mao Y, Sun F, et al. Production process and performance of recycled A356 aluminum alloy. Spec Cast Nonferrous Alloy 2022;42(4): 450–4.