Supplement materials

# Deep Reinforcement Learning-based Multi-Objective Scheduling for Distributed Heterogeneous Hybrid Flow Shops with Blocking Constraints

Xueyan Sun a, Weiming Shen a,*, Jiaxin Fan a,*, Birgit Vogel-Heuser b, Fandi Bi b, Chunjiang Zhang a

a State Key Laboratory of Intelligent Manufacturing Equipment and Technology, Huazhong University of Science and Technology, Wuhan 430074, China

b Institute of Automation and Information Systems, Technical University of Munich, Garching 85748, Germany

* Corresponding authors.

E-mail addresses: wshen@ieee.org (W. Shen), fanjx@hust.edu.cn (J. Fan).

Part A. Mathematical model of the multi-objective DHHBFSP

This section establishes a mathematical model to explain the multi-objective DHHBFSP. The notations and mathematical model are as follows:

| Indices |  |
| --- | --- |
| i,i' | Indices of jobs , i,i'= 1, 2, ..., N. |
| j | Indices of operations and stages, j = 1, 2, ..., S. |
| f | Indices of factories, f = 1, 2, ..., F. |
| k | Indices of machines, k = 1, 2, ..., Mf,j. |

| Parameters |  |
| --- | --- |
| N | Total number of jobs. |
| S | Total number of stages. |
| F | Total number of factories. |
| pi,j | Standard processing time of Ji at the jth stage. |
| Mf,j | Number of machines at the jth stage in factory Hf. |
| Di | The due date of Ji. |
| μf,j | Process time factor of operations at the jth stage in factories Hf. |
| ef,j | Standard unit time energy consumption at the jth stage in factories Hf. |
| eidle | Factor of unit energy consumption during the idle time of machines. |
| eblock | Factor of unit energy consumption during the blocking time of machines. |
| Ω | A very large positive number. |

| Decision variables |  |
| --- | --- |
| ci,j | Completion time of the jthoperation of Ji. |
| di,j | Departure time of the jthoperation of Ji. |
| ti | Tardiness of Ji. |
| Ep | Energy consumption during the processing of all machines. |
| Ei | Energy consumption during the idle time of all machines. |
| Eb | Energy consumption during the blocking time of all machines. |
| αi,j | Idle time of Ji at the jth operation. |
| βi,j | Blocking time of Ji at the jth operation. |
| xi,f | xi,f=1, if Ji is assigned to Hf; xi,f=0, otherwise. |
| yi,j,f,k | yi,j,f,k=1, if Ji is processed by the kth machine at the jth stage of Hf; yi,j,f,k=0, otherwise. |
| zi,i',j,f | zi,i',j,f=1, if Ji is processed after Ji' at the jthstage of Hf; zi,i',j,f=0, otherwise. |

Objectives:

Minmize TT=i=1Nti

Minimize TEC=Ep+Ei+Eb

s.t.

ti≥ci,S-Di,   ∀i          	(1)

ti≥0,   ∀i        	 (2)

Constraints (1) and (2) calculate the tardiness for all jobs.

Ep=i=1Nj=1Sf=1Fzi,f∙pi,j∙μf,j∙ef,j  	(3)

Ei=i=1Nj=1Sf=1Fxi,f∙αi,j∙ef,j∙eidle 	(4)

Eb=i=1Nj=1Sf=1Fxi,f∙βi,j∙ef,j∙eblock 	(5)

Equations (3), (4), (5) calculate the energy consumption during the processing, idle, and blocking states, respectively.

f=1Fxi,f=1，∀i                                           	   (6)

Constraint (6) ensures a job is assigned to only one factory.

k=1Mf,jyi,j,f,k=xi,f,  ∀i,j,f                                   	(7)

Constraint (7) ensures that a job selects an available machine from the assigned factory.

ci,j≥di',j+pi,j∙μf,j-3-yi,j,f,k-yi',j,f,k-zi,i',j,f∙ Ω,  ∀i,i',j,f,k  	(8)

ci',j≥di,j+pi',j∙μf,j-2-yi,j,f,k-yi',j,f,k+zi,i',j,f∙ Ω,  ∀i,i',j,f,k 	(9)

Constraints (8) and (9) prevent processing overlap between adjacent jobs on the same machine.

ci,j≥di,j-1+pi,j∙μf,j-1-yi,j,f,k∙ Ω,  ∀i,j>1,f,k 	(10)

Constraint (10) indicates that a job cannot be processed until it is released from the previous stage.

di,j≥ci,j, ∀i,j                                                              	 (11)

di,j≥di',j+1-3-yi,j+1,f,k-yi',j+1,f,k+zi,i',j+1,f∙ Ω,  ∀i,i',j<S,f,k 	(12)

Constraints (11) and (12) describe the blocking constraint, which indicates that a job can only be released when the operation is ﬁnished and there is an available machine at the next stage; otherwise, it should be held at the current machine.

Part B. Description of APN

APN [1] mainly consists of a feature extraction layer, query glimpse layer, and pointer layer, as shown in Figure S1. Before state feature extraction, the state feature vector [s1, s2, …, s8] of each agent is passed through a fully connected layer to form the job state feature vector.

To extract useful factory information for the optimization objective, the APN uses the attention layer in the transformer [2] to extract the feature eoutput, which can significantly reduce the processing time of the input sequence and increase the training speed of the entire policy. The attention layer builds dependencies between the input and output information entirely using a multi-head self-attention mechanism that contains two main sublayers: a multi-head self-attention layer and a feedforward network layer, each of which uses batch normalization (BN) to deal with outputs. To prevent gradient disappearance, residual connections between the sublayers were applied.

In the query glimpse layer, the attention mechanism uses multiple query vectors for eoutput. Each query vector focuses on a different part of the eoutput information during the query process, allowing the current eoutput information to be parsed from a different perspective. The obtained global shop floor vector is also used as a query vector to update the query vector again with each of the input job features eoutput to select the job that is most beneficial to the optimization objective.

Similar to the attention mechanism, a pointer mechanism [3] is used to generate a probability distribution for selecting the next job for processing by computing the relation values between the job feature vector eoutput and the query vector. A mask is a vector indicating the validation of actions. It is the same size as the actions. If action ai is valid, the mask mi is 1; otherwise, mi is 0.

Figure S1. APN structure

This study also uses the APN structure for critics, which contains a feature extraction layer, a query glimpse layer, and a Q-net layer. The Q-net layer is shown on the right side of Figure S1. Substituting the pointer with a Q-net can establish the APN structure in the critic. The output of the critic is the vector [q1, q2].

Part C. Response values of four parameters on different levels

Response values and rank of each parameter in parameter tuning are shown in Table S1.

Table S1. Response value and rank of each parameter

| level | B | γ | PR | TR |
| --- | --- | --- | --- | --- |
| 1 | 2004.772 | 1037.677 | 2146.401 | 484.269 |
| 2 | 646.074 | 904.923 | 1258.204 | 979.019 |
| 3 | 484.268 | 484.267 | 1031.888 | 1173.208 |
| Dalta | 1180.985 | 408.589 | 833.117 | 502.370 |
| rank | 1 | 4 | 2 | 3 |

Part D. IGD and P values of IPPO, PPO_S, PPO_OL & PPO_U

IGDs and P values of each instance for IPPO, PPO_S, PPO_OL and PPO_U with two different DDT can be found from Table S2 and Table S4.

Table S2. IGD values and P values between IPPO and the PPO_S

| Instances | DDT = 1.0 |  |  |  | DDT = 1.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IGD |  | P |  | IGD |  | P |  |
|  | PPO_S | IPPO | PPO_S | IPPO | PPO_S | IPPO | PPO_S | IPPO |
| J10E1 | 7.29E+03 | 4.54E+01 | 0.00 | 0.67 | 3.77E+03 | 0.00E+00 | 0.00 | 0.91 |
| J10E2 | 5.44E+03 | 1.60E+03 | 0.00 | 0.50 | 7.56E+03 | 0.00E+00 | 0.00 | 0.88 |
| J10E3 | 4.02E+03 | 3.11E+02 | 0.00 | 0.50 | 6.19E+03 | 1.66E+02 | 0.00 | 0.50 |
| J10E4 | 7.41E+03 | 3.38E+02 | 0.00 | 0.50 | 5.78E+03 | 1.50E+02 | 0.00 | 0.60 |
| J10E5 | 3.87E+03 | 4.90E+02 | 0.00 | 0.40 | 4.22E+03 | 1.78E+02 | 0.00 | 0.50 |
| J20E1 | 9.29E+03 | 0.00E+00 | 0.00 | 1.00 | 9.66E+03 | 6.33E+02 | 0.00 | 0.67 |
| J20E2 | 8.11E+03 | 3.78E+02 | 0.00 | 0.50 | 3.78E+03 | 4.83E+02 | 0.00 | 0.75 |
| J20E3 | 6.68E+03 | 0.00E+00 | 0.00 | 1.00 | 3.16E+03 | 0.00E+00 | 0.00 | 1.00 |
| J20E4 | 5.83E+03 | 3.98E+02 | 0.00 | 0.67 | 5.96E+03 | 0.00E+00 | 0.00 | 1.00 |
| J20E5 | 4.94E+03 | 7.45E+02 | 0.00 | 0.60 | 4.82E+03 | 1.22E+03 | 0.00 | 0.17 |
| J50E1 | 2.18E+03 | 8.07E+02 | 0.00 | 0.50 | 8.23E+03 | 9.08E+02 | 0.00 | 0.50 |
| J50E2 | 4.43E+03 | 9.16E+02 | 0.00 | 0.50 | 6.54E+03 | 0.00E+00 | 0.00 | 1.00 |
| J50E3 | 4.33E+03 | 4.63E+02 | 0.14 | 0.57 | 9.04E+03 | 0.00E+00 | 0.00 | 1.00 |
| J50E4 | 6.84E+03 | 2.11E+02 | 0.00 | 0.75 | 1.22E+04 | 6.87E+02 | 0.00 | 0.67 |
| J50E5 | 4.11E+03 | 1.56E+03 | 0.00 | 0.33 | 8.88E+03 | 0.00E+00 | 0.00 | 1.00 |
| J100E1 | 2.69E+03 | 3.07E+03 | 0.33 | 0.33 | 1.23E+04 | 7.35E+03 | 0.00 | 0.25 |
| J100E2 | 5.86E+03 | 2.14E+03 | 0.00 | 0.33 | 1.93E+04 | 0.00E+00 | 0.00 | 1.00 |
| J100E3 | 8.97E+03 | 1.11E+03 | 0.20 | 0.60 | 1.09E+04 | 0.00E+00 | 0.00 | 1.00 |
| J100E4 | 5.00E+03 | 4.54E+03 | 0.10 | 0.20 | 7.75E+03 | 3.21E+03 | 0.00 | 0.67 |
| J100E5 | 2.30E+03 | 3.40E+03 | 0.50 | 0.33 | 1.41E+04 | 2.78E+03 | 0.00 | 0.70 |
| J200E1 | 2.40E+04 | 8.64E+03 | 0.20 | 0.50 | 7.22E+04 | 3.56E+03 | 0.00 | 0.67 |
| J200E2 | 1.66E+04 | 1.56E+04 | 0.33 | 0.42 | 4.43E+04 | 4.56E+03 | 0.00 | 0.83 |
| J200E3 | 3.25E+04 | 1.86E+04 | 0.20 | 0.60 | 5.80E+04 | 4.85E+03 | 0.00 | 0.75 |
| J200E4 | 2.15E+04 | 1.34E+04 | 0.19 | 0.54 | 1.07E+04 | 2.60E+03 | 0.25 | 0.75 |
| J200E5 | 1.47E+04 | 1.23E+04 | 0.00 | 0.83 | 2.96E+05 | 3.28E+03 | 0.00 | 0.71 |

Table S3. IGD values and P values between IPPO and the PPO_OL

| Instances | DDT = 1.0 |  |  |  | DDT = 1.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IGD |  | P value |  | IGD |  | P value |  |
|  | PPO_OL | IPPO | PPO_OL | IPPO | PPO_OL | IPPO | PPO_OL | IPPO |
| J10E1 | 4.50E+02 | 4.54E+01 | 0.33 | 0.67 | 1.02E+03 | 0.00E+00 | 0.09 | 0.91 |
| J10E2 | 1.66E+03 | 1.60E+03 | 0.50 | 0.50 | 3.69E+03 | 0.00E+00 | 0.00 | 0.88 |
| J10E3 | 7.03E+02 | 3.11E+02 | 0.20 | 0.50 | 1.23E+03 | 1.66E+02 | 0.10 | 0.50 |
| J10E4 | 2.06E+02 | 3.38E+02 | 0.25 | 0.50 | 4.78E+03 | 1.50E+02 | 0.00 | 0.60 |
| J10E5 | 4.52E+02 | 4.90E+02 | 0.20 | 0.40 | 1.32E+03 | 1.78E+02 | 0.20 | 0.50 |
| J20E1 | 2.85E+03 | 0.00E+00 | 0.00 | 1.00 | 3.87E+03 | 6.33E+02 | 0.11 | 0.67 |
| J20E2 | 3.20E+02 | 3.78E+02 | 0.30 | 0.50 | 2.44E+03 | 4.83E+02 | 0.00 | 0.75 |
| J20E3 | 1.64E+03 | 0.00E+00 | 0.00 | 1.00 | 1.38E+03 | 0.00E+00 | 0.00 | 1.00 |
| J20E4 | 1.10E+03 | 3.98E+02 | 0.00 | 0.67 | 3.19E+03 | 0.00E+00 | 0.00 | 1.00 |
| J20E5 | 4.11E+03 | 7.45E+02 | 0.00 | 0.60 | 5.51E+02 | 1.22E+03 | 0.67 | 0.17 |
| J50E1 | 1.77E+03 | 8.07E+02 | 0.17 | 0.50 | 1.07E+03 | 9.08E+02 | 0.50 | 0.50 |
| J50E2 | 1.72E+03 | 9.16E+02 | 0.25 | 0.50 | 2.33E+03 | 0.00E+00 | 0.00 | 1.00 |
| J50E3 | 1.55E+03 | 4.63E+02 | 0.29 | 0.57 | 6.65E+03 | 0.00E+00 | 0.00 | 1.00 |
| J50E4 | 2.10E+03 | 2.11E+02 | 0.25 | 0.75 | 2.08E+03 | 6.87E+02 | 0.00 | 0.67 |
| J50E5 | 2.30E+03 | 1.56E+03 | 0.33 | 0.33 | 3.37E+03 | 0.00E+00 | 0.00 | 1.00 |
| J100E1 | 3.61E+03 | 3.07E+03 | 0.17 | 0.33 | 2.83E+02 | 7.35E+03 | 0.75 | 0.25 |
| J100E2 | 2.48E+03 | 2.14E+03 | 0.33 | 0.33 | 1.05E+04 | 0.00E+00 | 0.00 | 1.00 |
| J100E3 | 5.39E+03 | 1.11E+03 | 0.20 | 0.60 | 9.77E+03 | 0.00E+00 | 0.00 | 1.00 |
| J100E4 | 9.60E+03 | 4.54E+03 | 0.00 | 0.20 | 8.07E+03 | 3.21E+03 | 0.00 | 0.67 |
| J100E5 | 5.20E+03 | 3.40E+03 | 0.17 | 0.33 | 7.11E+03 | 2.78E+03 | 0.00 | 0.70 |
| J200E1 | 2.50E+04 | 8.64E+03 | 0.30 | 0.50 | 8.11E+03 | 3.56E+03 | 0.33 | 0.67 |
| J200E2 | 1.78E+04 | 1.56E+04 | 0.25 | 0.42 | 3.33E+04 | 4.56E+03 | 0.00 | 0.83 |
| J200E3 | 5.00E+04 | 1.86E+04 | 0.00 | 0.60 | 1.12E+04 | 4.85E+03 | 0.25 | 0.75 |
| J200E4 | 2.67E+04 | 1.34E+04 | 0.19 | 0.54 | 1.27E+04 | 2.60E+03 | 0.00 | 0.75 |
| J200E5 | 1.60E+04 | 1.23E+04 | 0.17 | 0.83 | 2.19E+04 | 3.28E+03 | 0.14 | 0.71 |

Table S4. IGD values and P values between IPPO and the PPO_U

| Instances | DDT = 1.0 |  |  |  | DDT = 1.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IGD |  | P |  | IGD |  | P |  |
|  | PPO_U | IPPO | PPO_U | IPPO | PPO_U | IPPO | PPO_U | IPPO |
| J10E1 | 2.58E+03 | 4.54E+01 | 0.00 | 0.67 | 6.01E+03 | 0.00E+00 | 0.00 | 0.91 |
| J10E2 | 2.12E+03 | 1.60E+03 | 0.00 | 0.50 | 1.29E+03 | 0.00E+00 | 0.13 | 0.88 |
| J10E3 | 5.58E+02 | 3.11E+02 | 0.30 | 0.50 | 3.98E+02 | 1.66E+02 | 0.40 | 0.50 |
| J10E4 | 6.66E+02 | 3.38E+02 | 0.25 | 0.50 | 4.62E+02 | 1.50E+02 | 0.40 | 0.60 |
| J10E5 | 6.64E+02 | 4.90E+02 | 0.40 | 0.40 | 4.31E+02 | 1.78E+02 | 0.30 | 0.50 |
| J20E1 | 1.57E+03 | 0.00E+00 | 0.00 | 1.00 | 6.73E+02 | 6.33E+02 | 0.22 | 0.67 |
| J20E2 | 1.11E+03 | 3.78E+02 | 0.20 | 0.50 | 2.13E+03 | 4.83E+02 | 0.25 | 0.75 |
| J20E3 | 3.85E+03 | 0.00E+00 | 0.00 | 1.00 | 2.77E+03 | 0.00E+00 | 0.00 | 1.00 |
| J20E4 | 1.05E+03 | 3.98E+02 | 0.33 | 0.67 | 1.62E+03 | 0.00E+00 | 0.00 | 1.00 |
| J20E5 | 2.74E+03 | 7.45E+02 | 0.40 | 0.60 | 6.64E+02 | 1.22E+03 | 0.17 | 0.17 |
| J50E1 | 1.52E+03 | 8.07E+02 | 0.33 | 0.50 | 4.09E+03 | 9.08E+02 | 0.00 | 0.50 |
| J50E2 | 1.56E+03 | 9.16E+02 | 0.25 | 0.50 | 4.38E+03 | 0.00E+00 | 0.00 | 1.00 |
| J50E3 | 3.09E+03 | 4.63E+02 | 0.00 | 0.57 | 6.58E+03 | 0.00E+00 | 0.00 | 1.00 |
| J50E4 | 4.28E+03 | 2.11E+02 | 0.00 | 0.75 | 1.42E+03 | 6.87E+02 | 0.33 | 0.67 |
| J50E5 | 3.12E+03 | 1.56E+03 | 0.33 | 0.33 | 3.17E+03 | 0.00E+00 | 0.00 | 1.00 |
| J100E1 | 8.19E+03 | 3.07E+03 | 0.17 | 0.33 | 1.12E+04 | 7.35E+03 | 0.00 | 0.25 |
| J100E2 | 1.58E+03 | 2.14E+03 | 0.33 | 0.33 | 7.99E+03 | 0.00E+00 | 0.00 | 1.00 |
| J100E3 | 1.33E+04 | 1.11E+03 | 0.00 | 0.60 | 7.28E+03 | 0.00E+00 | 0.00 | 1.00 |
| J100E4 | 4.17E+03 | 4.54E+03 | 0.20 | 0.20 | 5.13E+03 | 3.21E+03 | 0.33 | 0.67 |
| J100E5 | 5.27E+03 | 3.40E+03 | 0.00 | 0.33 | 5.90E+03 | 2.78E+03 | 0.30 | 0.70 |
| J200E1 | 4.35E+04 | 8.64E+03 | 0.00 | 0.50 | 9.36E+03 | 3.56E+03 | 0.00 | 0.67 |
| J200E2 | 2.28E+04 | 1.56E+04 | 0.00 | 0.42 | 3.61E+04 | 4.56E+03 | 0.17 | 0.83 |
| J200E3 | 2.38E+04 | 1.86E+04 | 0.20 | 0.60 | 2.71E+04 | 4.85E+03 | 0.00 | 0.75 |
| J200E4 | 1.53E+04 | 1.34E+04 | 0.15 | 0.54 | 1.49E+05 | 2.60E+03 | 0.00 | 0.75 |
| J200E5 | 3.61E+04 | 1.23E+04 | 0.00 | 0.83 | 2.38E+04 | 3.28E+03 | 0.14 | 0.71 |

Part E. Detailed comparison between IPPO and PPO_S, PPO_OL & PPO_U

To demonstrate the trend in the change in the number of jobs with different DDT values, we calculated the average values of IGD and P with different numbers of jobs. Figure S2 shows that with an increase in the number of jobs, the IGD of all methods increases regardless of whether the DDT values change, which is caused by the scale of the instances. However, the methods do not exhibit similar trends in P values with an increasing number of jobs. We can learn from the right side of Figure S4 that all methods change slightly in the P value as the number of jobs increases when the DDT value is 1.5. However, with a DDT of 1.0, as shown on the left side of Figure S3, with the exception of the PPO_U, the other methods were unstable. However, with DDT = 1.0, IPPO performed the best with 20 jobs. This is because the model was trained for each episode with the insertion of 20 jobs and a DDT of 1.5. Therefore, the proposed IPPO performed better than the other unimproved methods, although it was sensitive to the training setting.

Figure S2. IGD value trend of different methods vs. number of jobs

Figure S3. P value trend of different methods vs. number of jobs

Part F. IGD and P values of IPPO, dispatching rules, metaheuristics & DRL methods

IGDs and P values of each instance for IPPO, dispatching rules containing earliest due date (EDD), modified due date (MDD), minimum slack time (MST), shortest process time (SPT), and largest process time (LPT), and one commonly used MODRL method (abbreviated as SW) can be found in Table S5 and Table S6.

Table S5. IGD values between IPPO and other rules

| Instances | EDD | MDD | MST | LPT | SPT | SW | IPPO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| J10E1 | 2.18E+04 | 1.08E+04 | 1.73E+04 | 2.53E+04 | 1.33E+04 | 1.53E+04 | 0.00E+00 |
| J10E2 | 1.94E+04 | 9.42E+03 | 1.80E+04 | 2.40E+04 | 1.30E+04 | 1.40E+04 | 1.63E+03 |
| J10E3 | 2.29E+04 | 2.19E+04 | 1.73E+04 | 2.41E+04 | 1.62E+04 | 1.81E+04 | 0.00E+00 |
| J10E4 | 2.09E+04 | 1.95E+04 | 1.62E+04 | 2.62E+04 | 1.42E+04 | 1.62E+04 | 0.00E+00 |
| J10E5 | 1.96E+04 | 1.73E+04 | 1.84E+04 | 2.15E+04 | 1.58E+04 | 1.13E+04 | 0.00E+00 |
| J20E1 | 2.36E+04 | 1.23E+04 | 1.80E+04 | 2.34E+04 | 1.63E+04 | 1.44E+04 | 0.00E+00 |
| J20E2 | 2.26E+04 | 1.36E+04 | 1.69E+04 | 2.01E+04 | 1.79E+04 | 1.01E+04 | 0.00E+00 |
| J20E3 | 2.56E+04 | 1.04E+04 | 1.68E+04 | 1.83E+04 | 1.62E+04 | 1.53E+04 | 0.00E+00 |
| J20E4 | 2.28E+04 | 1.10E+04 | 1.86E+04 | 2.41E+04 | 1.85E+04 | 1.41E+04 | 0.00E+00 |
| J20E5 | 2.60E+04 | 1.22E+04 | 1.56E+04 | 1.99E+04 | 1.79E+04 | 1.27E+04 | 0.00E+00 |
| J50E1 | 3.03E+04 | 3.10E+04 | 3.08E+04 | 1.81E+04 | 3.08E+04 | 1.97E+04 | 7.50E+03 |
| J50E2 | 2.71E+04 | 3.21E+04 | 3.19E+04 | 2.46E+04 | 2.19E+04 | 2.46E+04 | 1.17E+04 |
| J50E3 | 2.39E+04 | 3.88E+04 | 2.88E+04 | 2.17E+04 | 2.88E+04 | 2.32E+04 | 0.00E+00 |
| J50E4 | 2.43E+04 | 3.72E+04 | 2.81E+04 | 2.22E+04 | 2.41E+04 | 1.93E+04 | 9.21E+03 |
| J50E5 | 2.28E+04 | 4.01E+04 | 2.63E+04 | 2.19E+04 | 2.33E+04 | 2.20E+04 | 4.19E+03 |
| J100E1 | 1.65E+05 | 8.29E+04 | 6.45E+04 | 8.12E+04 | 9.15E+04 | 4.16E+04 | 2.38E+04 |
| J100E2 | 1.52E+05 | 1.03E+05 | 6.55E+04 | 8.76E+04 | 8.47E+04 | 4.25E+04 | 1.16E+04 |
| J100E3 | 9.30E+04 | 9.41E+04 | 8.18E+04 | 9.12E+04 | 9.28E+04 | 4.19E+04 | 5.93E+03 |
| J100E4 | 1.01E+05 | 9.68E+04 | 7.35E+04 | 8.34E+04 | 9.04E+04 | 4.64E+04 | 3.85E+04 |
| J100E5 | 1.35E+05 | 7.83E+04 | 9.13E+04 | 8.92E+04 | 8.93E+04 | 4.51E+04 | 5.81E+04 |
| J200E1 | 2.91E+05 | 2.83E+05 | 3.05E+05 | 2.21E+05 | 2.31E+05 | 7.04E+04 | 0.00E+00 |
| J200E2 | 3.24E+05 | 2.42E+05 | 3.41E+05 | 2.26E+05 | 2.67E+05 | 7.26E+04 | 0.00E+00 |
| J200E3 | 3.51E+05 | 2.79E+05 | 3.31E+05 | 2.33E+05 | 2.33E+05 | 6.75E+04 | 2.20E+05 |
| J200E4 | 3.00E+05 | 2.53E+05 | 3.19E+05 | 2.15E+05 | 2.19E+05 | 7.33E+04 | 1.23E+04 |
| J200E5 | 3.11E+05 | 2.66E+05 | 3.10E+05 | 2.03E+05 | 2.66E+05 | 7.72E+04 | 0.00E+00 |

Table S6. P values between IPPO and other rules

| Instances | EDD | MDD | MST | LPT | SPT | SW | IPPO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| J10E1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J10E2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.50 |
| J10E3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J10E4 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J10E5 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J20E1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J20E2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J20E3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J20E4 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J20E5 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J50E1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.50 |
| J50E2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.33 |
| J50E3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J50E4 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.50 |
| J50E5 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J100E1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| J100E2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J100E3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.33 |
| J100E4 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.33 |
| J100E5 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J200E1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J200E2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J200E3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J200E4 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.33 |
| J200E5 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |

IGDs and P values of each instance for IPPO, metaheuristics (MOEAD23 and Pdjaya23) and DRL methods (MAEQL and MAGPI) can be found in Table S7 and Table S8.

Table S7. IGD values between IPPO and other algorithms

| Instances | MOEAD23 | Pdjaya23 | MAEQL | MAGPI | IPPO |
| --- | --- | --- | --- | --- | --- |
| J10E1 | 4.77E+03 | 3.84E+03 | 8.28E+03 | 5.34E+03 | 0.00E+00 |
| J10E2 | 4.45E+03 | 5.42E+03 | 7.99E+03 | 4.01E+03 | 1.63E+03 |
| J10E3 | 2.54E+04 | 2.19E+04 | 2.73E+04 | 2.01E+04 | 0.00E+00 |
| J10E4 | 2.09E+04 | 2.05E+04 | 3.42E+04 | 2.62E+04 | 0.00E+00 |
| J10E5 | 1.86E+04 | 1.73E+04 | 2.88E+04 | 2.10E+04 | 0.00E+00 |
| J20E1 | 1.36E+04 | 1.27E+04 | 3.36E+04 | 1.64E+04 | 0.00E+00 |
| J20E2 | 1.21E+04 | 1.43E+04 | 2.29E+04 | 1.01E+04 | 0.00E+00 |
| J20E3 | 1.36E+04 | 1.74E+04 | 3.12E+04 | 1.63E+04 | 0.00E+00 |
| J20E4 | 1.68E+04 | 1.10E+04 | 2.30E+04 | 7.14E+03 | 0.00E+00 |
| J20E5 | 1.30E+04 | 1.82E+04 | 2.94E+04 | 9.93E+03 | 0.00E+00 |
| J50E1 | 1.13E+04 | 1.10E+04 | 4.08E+04 | 8.15E+03 | 7.50E+03 |
| J50E2 | 1.71E+04 | 1.42E+04 | 4.19E+04 | 4.60E+03 | 1.17E+04 |
| J50E3 | 2.07E+04 | 1.90E+04 | 2.88E+04 | 2.32E+04 | 0.00E+00 |
| J50E4 | 1.43E+04 | 1.12E+04 | 3.81E+04 | 1.42E+04 | 9.21E+03 |
| J50E5 | 2.76E+04 | 1.47E+04 | 3.63E+04 | 2.00E+04 | 4.19E+03 |
| J100E1 | 6.78E+04 | 8.46E+04 | 3.15E+04 | 2.16E+04 | 2.38E+04 |
| J100E2 | 7.23E+04 | 6.08E+04 | 2.41E+04 | 0.00E+00 | 1.16E+04 |
| J100E3 | 8.30E+04 | 7.71E+04 | 3.28E+04 | 2.25E+04 | 5.93E+03 |
| J100E4 | 7.14E+04 | 4.62E+04 | 4.15E+04 | 2.64E+04 | 3.85E+04 |
| J100E5 | 6.53E+04 | 8.25E+04 | 1.03E+05 | 9.81E+04 | 5.81E+04 |
| J200E1 | 2.41E+05 | 2.03E+05 | 1.83E+05 | 2.05E+05 | 0.00E+00 |
| J200E2 | 1.17E+05 | 1.30E+05 | 5.12E+04 | 2.26E+04 | 0.00E+00 |
| J200E3 | 4.51E+05 | 1.33E+05 | 5.13E+04 | 3.29E+04 | 2.20E+05 |
| J200E4 | 2.40E+04 | 1.29E+05 | 3.65E+04 | 5.33E+03 | 1.23E+04 |
| J200E5 | 1.11E+05 | 1.56E+05 | 1.85E+04 | 1.22E+04 | 0.00E+00 |

Table S8. P values between IPPO and other algorithms

| Instances | MOEAD23 | Pdjaya23 | MAEQL | MAGPI | IPPO |
| --- | --- | --- | --- | --- | --- |
| J10E1 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J10E2 | 0.25 | 0.25 | 0.00 | 0.00 | 0.50 |
| J10E3 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J10E4 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J10E5 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J20E1 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J20E2 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J20E3 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J20E4 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J20E5 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J50E1 | 0.00 | 0.00 | 0.00 | 0.50 | 0.50 |
| J50E2 | 0.00 | 0.00 | 0.00 | 0.67 | 0.33 |
| J50E3 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J50E4 | 0.00 | 0.00 | 0.00 | 0.50 | 0.50 |
| J50E5 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J100E1 | 0.50 | 0.00 | 0.00 | 0.50 | 0.00 |
| J100E2 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J100E3 | 0.00 | 0.00 | 0.00 | 0.67 | 0.33 |
| J100E4 | 0.00 | 0.00 | 0.00 | 0.67 | 0.33 |
| J100E5 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J200E1 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J200E2 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J200E3 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |
| J200E4 | 0.00 | 0.00 | 0.00 | 0.67 | 0.33 |
| J200E5 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |

References

[1]	L. Zhao, J. Fan, C. Zhang, W. Shen, and J. Zhuang, "A DRL-Based Reactive Scheduling Policy for Flexible Job Shops With Random Job Arrivals," IEEE Transactions on Automation Science and Engineering, 2023.

[2]	A. Vaswani et al., "Attention is all you need," Advances in neural information processing systems, vol. 30, 2017.

[3]	O. Vinyals, M. Fortunato, and N. Jaitly, "Pointer networks," Advances in neural information processing systems, vol. 28, 2015.