Supplementary data for

# CO2 Utilization and Geological Storage in Unconventional Reservoirs After Fracturing

Jinzhou Zhao a, Lele Wang a, Bing Wei a,*, Valeriy Kadet b

a State Key Laboratory of Oil and Gas Reservoir Geology and Exploitation, Southwest Petroleum University, Chengdu 610500, China

b Gubkin Russian State University of Oil and Gas, Moscow 119991, Russia

* Corresponding authors.

E-mail address: bwei@swpu.edu.cn (B. Wei)

Table S1

Agreement between measured data and simulated results.

| Parameter | Measured | Simulated |
| --- | --- | --- |
| Saturation pressure (MPa) | 22.00 | 21.88 |
| Average GOR (m3/m3) | 108 | 103.97 |
| Crude oil viscosity (cp) | 0.41 | 0.42 |
| MMP of CO2 (MPa) | 33.6 | 34 |

Fig. S1. Pressure-dependent porosity-permeability multiplier curves, (a) Matrix, and (b) Fracture.

Table S2

Chemical equilibrium coefficients of aqueous and mineral reactions.

| Reactions |  | a0 | a1 | a2 | a3 | a4 |
| --- | --- | --- | --- | --- | --- | --- |
| Aqueous reactions | CO2(aq) + H2O ⇌ H+ + HCO3- | -6.5492 | 0.0090 | -1.021E-04 | 2.762E-07 | -3.561E-10 |
|  | CO32- + H+ ⇌ HCO3- | 10.6080 | -0.0128 | 1.203E-04 | -3.017E-07 | 2.694E-10 |
|  | H+ + OH- ⇌ H2O | 14.9282 | -0.0419 | 1.974E-04 | -5.550E-07 | 7.581E-10 |
| Mineral reactions | NaAlSi3O8+4H+ ⇌ Al3++2H2O+Na++3SiO2(aq) | 3.9232 | -0.0345 | 2.524E-05 | 3.099E-07 | -8.314E-10 |
|  | CaAl2Si2O8+8H+ ⇌ 2Al3++Ca2++4H2O+2SiO2(aq) | 31.7457 | -0.2013 | 5.959E-04 | -9.041E-07 | 9.154E-11 |
|  | K0.6Mg0.25Al2.3Si3.5O10(OH)2+8H+⇌2.3Al3++5H2O+0.6K++0.25Mg2++3.5SiO2(aq) | 12.4348 | -0.1123 | 2.717E-04 | -8.056E-08 | -8.338E-10 |
|  | KAlSi3O8+4H+ ⇌ Al3++2H2O+K++3SiO2(aq) | 0.4613 | -0.0151 | -3.892E-05 | 4.290E-07 | -9.219E-10 |
|  | SiO2 ⇌ SiO2(aq) | -4.4970 | 0.0220 | -1.073E-04 | 3.246E-07 | -3.967E-10 |
|  | Mg5Al2Si3O10(OH)8+16H+ ⇌ 2Al3++12H2O+5Mg2++3SiO2(aq) | 78.2910 | -0.4198 | 1.302E-03 | -2.107E-06 | 5.405E-10 |
|  | Al2Si2O5(OH)4+6H+ ⇌ 2Al3++5H2O+2SiO2(aq) | 9.7295 | -0.0989 | 2.916E-04 | -3.270E-07 | -3.311E-10 |
|  | CaCO3+H+ ⇌ Ca2++HCO3- | 2.0689 | -0.0143 | -6.061E-06 | 1.459E-07 | -4.189E-10 |
|  | MgCO3+H+ ⇌ HCO3-+Mg2+ | 3.1181 | -0.0284 | 4.142E-05 | 4.757E-08 | -3.488E-10 |
|  | CaSO4 ⇌ Ca2++SO42- | -4.0691 | -0.0057 | -1.102E-04 | 4.993E-07 | -9.869E-10 |
|  | CaMg(CO3)2+2H+ ⇌ Ca2++2HCO3-+Mg2+ | 3.3944 | -0.0356 | 1.326E-05 | 2.411E-07 | -8.149E-10 |
|  | NaAlCO3(OH)2+3H+ ⇌ Al3++2H2O+HCO3-+Na+ | 5.8195 | -0.0508 | 2.213E-04 | -5.232E-07 | 4.089E-10 |

Fig. S2. The change region in CO2 mole fraction in the oil and water phases in the first soaking stage. The CO2 mole fraction change was less than -10-3 for release and greater than 10-3 for dissolution, (a) CO2 release zone in the oil phase; (b) CO2 dissolution zone in the oil phase; (c) CO2 release zone in the water phase; (d) CO2 dissolution zone in the water phase.