Supplementary data for

# Optimal Scheduling and On-the-Fly Flexible Control of Integrated Energy Systems for Residential Buildings Considering Photovoltaic Prediction Errors

Ziqing Wei a, Xiaoqiang Zhai a,*, Ruzhu Wang a,*

a Institute of Refrigeration and Cryogenics, Shanghai Jiao Tong University, Shanghai 200240, China

* Corresponding authors.

E-mail addresses: xqzhai@sjtu.edu.cn (X. Zhai), rzwang@sjtu.edu.cn (R. Wang).

## Section 1: CNN-LSTM structure

Table S1 outlies the CNN-LSTM model’s architecture, which is composed of 1D-CNN, LSTM, and fully connected (FC) layers. The CNN and LSTM work together to extract important features using an end-to-end approach, eliminating the need for manual feature extraction. The model is implemented through PyTorch software on hardware equipped with i7-7700HQ@3.4GHz, 24 G RAM, and GTX1060 with CUDA.

Table S1 Construction of CNN-LSTM model

| Layer Number | Type | Unit | Activate function |
| --- | --- | --- | --- |
| 1 | Conv1D | 64 | ReLU |
| 2 | Conv1D | 64 | ReLU |
| 3 | Pool | - | - |
| 4 | Flatten | 128 | - |
| 5 | LSTM | 128 | ELU |
| 6 | LSTM | 128 | ELU |
| 7 | Dense | 128 | ReLU |
| 8 | Dense | 128 | ReLU |
| 9 | Dense | 128 | ReLU |
| 10 | Output | 96 | - |

Section 2: Convolution Neural Network layer

Two convolutional filters are used to extract the features among different sensor data. Alongside the FC layer, the primary components of a CNN include the convolutional layer, activation function, and pooling layer. The convolutional layer and activation function are usually combined and implemented as follows,

yi=ReluXi*W+θi=Reluj=1kxi+j-1wj+θiXi∈Χ,&Xi=[x1,x2,x3…,xn],&W=[w1,w2,w3…,xk]	(S1)

where yi represents the output of the i-th component from the convolution operation. Xi denotes the i-th 1D inputs of the total dataset X, which contains n features. W corresponds to the convolutional kernel that contains k elements. Symbol * represents the convolution action of dot product and summation. θi signifies the bias term, and Relu() denotes the activation function.

After applying convolution, pooling is applied to reduce the dimensionality of the feature maps, thereby reducing the computational complexity and controlling overfitting,

Zi=MAXyi,yi+1,...,yi+m-1	(S2)

where  is the i-th output of intermediate layer, MAX() is the function that takes the maximum value.

Section 3. Long Short-Term Memory layer

LSTM is a specific form of recurrent neural network (RNN), which is widely used for time series prediction. It addresses the gradient vanishing and exploding issues through gating mechanisms. The core components of an LSTM include the input gate, forget gate, cell state, and output gate.

In a sequence, the LSTM cell processes each step t by updating its hidden state ht and cell state ct based on the previous hidden state ht-1, cell state ct-1, and the current input xt. An LSTM cell can be expressed as follows,

Γτ=σΓτ=σ(WxΓxτ+WΠΓΠτ-1+θΓ)Λτ=σΛτ=σ(WxΛxτ+WΠΛΠτ-1+θi)Θτ=tanhΛτ=tanh(WxΘxτ+WΠΘΠτ-1+θΘ)Oτ=σOτ=σ(WxOxτ+WΠOΠτ-1+θO)Ψτ=Ψτ-1⊙Γτ+Θτ⊙ΛτΠτ=Oτ⊙tanh⁡(Ψτ)yτ=WyΠΠτ+θy	(S3)

where Γτ, Λτ, Θτ, Oτ, Ψτ, Πτ, and yτ represent the outputs of the forget gate, input gate, candidate layer for input gate, output gate, update cell state, hidden state, and final output of time slot τ, respectively. xτ is input at time step t, Πτ-1 is the hidden state at the previous time step τ-1, Ψτ-1 is cell state at the previous time step τ-1, WxΓ, WxΛ, WxΘ, WxO, and WyΠ are the weights for the gates (forget, input, candidate layer for input, output, and final output gate), θ denotes the biases for each gates, σ() and tanh() are the activation function of the sigmoid and hyperbolic tangent, respectively.

Section 4: Full connected layer

A full connected layer can be expressed as follows,

Yi=f(XiW+θi)	(S4)

where Yi is the i-th output, Xi represents the i-th input, W corresponds to the weight matrix, θi is the bias, and f() is the activation function.

## Section 5: Entropy weight method

According to Problem 1, two weights α1 and α2 are determined through the entropy weight method (EWM) which is based on information theory [45] to access data uncertainty or disorder. In the context of this bi-objective optimization problem, EWM can be applied to determine the relative importance of each objective. The pseudocode for calculating weights is shown in Algorithm 1.

Algorithm 1: Weights calculation based on EWM

Inputs: Model(m), weather_array([tambient, I]) Outputs: [α1, α2] Launch optimization model m Launch [tambient, I] # STEP 1 Set objective function Set object function 1 obj1 = Eq. (22) Set object function 2 obj2 = Eq. (23) # STEP 2 Get decision matrix Activate m.obj1, deactivate m.obj2 Initialize m, Solve m Get minimum Zeconomic, maximum Zcomfort Activate m.obj2, deactivate m.obj1 Solve m Get minimum Zcomfort, maximum Zeconomic obj_list = [] for in range(0, 1, 0.025): Set objective function 3 obj = λZeconomic +(1-λ)Zcomfort Solve m obj_list.append([Zeconomic, Zcomfort]) end for # STEP 3 Normalization obj1_min = np.array(obj_list).T[0].min() obj1_max = np.array(obj_list).T[0].max() obj2_min = np.array(obj_list).T[1].min() obj2_max = np.array(obj_list).T[1].max() Get the normalized data(norm_data) through Eq. (24) and Eq. (25) # STEP 4 Probability and Entropy m, n = norm_data.shape[0], norm_data.shape[1] k = 1/log(m) p = norm_data / np.sum(norm_data, axis=0) entro_mat = -k * np.sum(p * np.log(p + np.finfo(float).eps), axis=0) # STEP 5 Divergence and Entropy Weights d = 1 - entro_mat weights = d / np.sum(d) Return weights

The first step involves establishing two single objective functions based on Eqs. (22) and Eq. (23). In the second step, a temporary weight λ ranging from 0 to 1 in increments of 0.025 is applied to form the multi-objective function. A decision matrix that contains 40 sets of [obj1, obj2] can be obtained from various λ settings. The decision matrix shown in Fig. S1 illustrates the Pareto-front formed by the two objective functions. In the third step, the decision matrix is normalized through min-max scaling, as given in Eq. (24) and Eq. (25). The fourth step involves calculating the entropy matrix, where k is a constant decided by the number of rows of the decision matrix, and p denotes the proportion for each indicator. The entropy matrix is derived using the Shannon EWM formula [45]. In the final step, the difference coefficient d and the weights can be obtained. The weights of α1 and α2 are 0.14 and 0.86 for the case study, respectively.

Fig. S1. Results obtained for bi-objective optimization.

References

[45] Shannon CE. A mathematical theory of communication. Bell Syst Tech J 1948;27(3):379–423.