Supplementary data for

Emerging Organic Contaminants in Chinese Surface Water: Identification of Priority Pollutants

Mengmeng Zhonga, Tielong Wanga, Wenxing Zhaoa, Jun Huanga, Bin Wanga, Lee Blaneya,b, Qingwei Buc, Gang Yua,*

a School of Environment, Beijing Key Laboratory for Emerging Organic Contaminants Control, State Key Joint Laboratory of Environment Simulation and Pollution Control (SKLESPC), Tsinghua University, Beijing 100084, China

b Department of Chemical, Biochemical, and Environmental Engineering, University of Maryland Baltimore County, Baltimore, MD 21250, USA

c School of Chemical & Environmental Engineering, China University of Mining & Technology-Beijing, Beijing 100083, China

* Corresponding author.

E-mail address: yg-den@tsinghua.edu.cn (G. Yu)

Table S1.

Utility functions (U) used to evaluate the six prioritization criteria, hazard potential and exposure potential.

| Criteria | Utility functions | Example calculation using erythromycin |
| --- | --- | --- |
| Concentration | UM=log2(C)-log2(Cmin, 4)log2(Cmax)-log2(Cmin, 4), where C is the concentration of the EOC in water, Cmax is the maximum concentration in the overall list of candidate EOCs, and Cmin,4 is the fourth lowest EOC concentration from the overall list of candidate EOCs | U(F) = [log2 (119) - log2 (0.0067)]/[(log2 (3270) - log2 (0.0067)] = 0.75 |
| Detection frequency | U(F) = (f / 100), where f is the detection frequency of the EOC of concern | U(C) = 100/100 = 1 |
| Bioaccumulation | UB=log10Dow-log10Dowminlog10Dow,max,5-log10Dow,min, where Dow is the octanol-water partition coefficient for the EOC at pH 7, Dow,max,5 is the fifth highest Dow value in the overall list of candidate EOCs, and Dow,min is the lowest Dow value in the overall list of candidate EOCs | U(B) = [0.81 - (-5.46)]/[10.36 - (-5.46)] = 0.40 |
| Persistence | UP=1-H-Hmin, 31Hmax-Hmin,31, where H is the degradation half-life for the EOC in water, Hmin,31 is the 31st lowest degradation half-life from the overall list of candidate EOCs, and Hmax is the maximum degradation half-life from the overall list of candidate EOCs | U(P) = 1 - (1.24 - 1.02)/(3.66 - 1.02) = 0.91 |
| Ecotoxicity | UE=1-log10(PNECeco)-log10(PNECeco,min,4)log10(PNECeco,max,5)-log10(PNECeco,min,4), where PNECeco is the predicted no effect concentration for ecological effects for the EOC, PNECeco,max,5 is the fifth highest PNECeco value in the overall list of candidate EOCs, and PNECeco,min,4 is the fourth lowest PNECeco value in the overall list of candidate EOCs | U(E) =1 - [log10 (200) - log10 (0.01)]/ [log10 (2.28×108) - log10 (0.01)] = 0.58 |
| Human health effects | UH=1-log10(PNEChum)-log10(PNEChum,min,5)log10(PNEChum,max)-log10(PNEChum,min,5), where PNEChum is the predicted no effect concentration for human health effects for the EOC, PNEChum,max is the highest PNEChum value in the overall list of candidate EOCs, and PNEChea,min,5 is the fifth lowest PNEChum value in the overall list of candidate EOCs | U(H) =1 - [log10 (22660) - log10 (12)]/ [log10 (1.31×109) - log10 (12)] = 0.59 |
| Hazard potential | UHP=HP-HPminHPmax-HPmin, where HP is the hazard potential of the EOC, HPmax is the maximum HP in the overall list of candidate EOCs, HPmin is the lowest HP value in the overall list of candidate EOCs | U(HP) = [0.77-(-2.79)]/[2.61-(-2.79)] = 0.66 |
| Exposure potential | UEP=EP-EPminEPmax-EPmin, where EP is the hazard potential of the EOC, EPmax is the maximum EP in the overall list of candidate EOCs, EPmin is the lowest EP value in the overall list of candidate EOCs | U(EP) = [1.25-(-2.22)]/[1.89-(-2.22)] = 0.84 |

Table S2.

EOC occurrence data, including detection frequency and concentration.

| CAS | Compounds | Concentration |  |  |  | Frequency |  |  |  | Refs. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max. | Min. | Med. | 90% | Max. | Min. | Med. | 90% |  |
| 2634-33-5 | 1,2-Benzisothiazolinone | 7.5 | 0.04 | 1.9 | 5.4 | 100 | 0 | 79 | 96 | [1–5] |
| 27833-18-7 | 17α-Boldenone | 0.01 | 0.01 | 0.01 | 0.01 | 0 | 0 | 0 | 0 | [6] |
| 57-91-0 | 17α-Estradiol | 15 | 0 | 4.0 | 9.8 | 93 | 14 | 46 | 82 | [7–9] |
| 57-63-6 | 17α-Ethinyl estradiol | 207 | 0 | 2.9 | 22 | 100 | 0 | 19 | 100 | [7–23] |
| 68-96-2 | 17α-Hydroxyprogesterone | 0.007 | 0.007 | 0.007 | 0.007 | 2 | 2 | 2 | 2 | [24] |
| 80657-17-6 | 17α-Trenbolone | 0.04 | 0.04 | 0.04 | 0.04 | 0 | 0 | 0 | 0 | [6] |
| 50-28-2 | 17β-Estradiol | 59 | 0 | 5.6 | 28 | 100 | 0 | 6 | 100 | [6–12, 14, 15, 17, 19–28] |
| 10161-33-8 | 17β-Trenbolone | 0.06 | 0.06 | 0.06 | 0.06 | 0 | 0 | 0 | 0 | [6] |
| 120-95-6 | 2,4-Di-tert-pentylphenol | 4.9 | 4.9 | 4.9 | 4.9 | 100 | 100 | 100 | 100 | [29] |
| 1241-94-7 | 2-Ethylhexyl Diphenyl phosphate | 4.5 | 0.03 | 2.1 | 3.8 | 100 | 7 | 55 | 96 | [30–33] |
| 30223-73-5 | 2-Ethylidene -1,5-dimethyl-3,3-diphenylpyrrolidine | 0.01 | 0.01 | 0.01 | 0.02 | 2 | 2 | 2 | 8 | [34] |
| 90-43-7 | 2-Phenylphenol | 0.6 | 0.60 | 0.60 | 0.60 | 100 | 100 | 100 | 100 | [35] |
| 64359-81-5 | 4,5-Dichloro-2-octyl-isothiazolone | 0.05 | 0.05 | 0.05 | 0.05 | 15 | 15 | 15 | — | [2] |
| ’63-05-8 | 4-Androstene-3,17-dione | 0.02 | 0.02 | 0.02 | 0.02 | 11 | 11 | 11 | 11 | [6] |
| 1638-22-8 | 4-Butylphenol | 57 | 10 | 33 | 52 | 100 | 41 | 70 | 94 | [29, 36] |
| 79-85-6 | 4-Epitetracycline | 2.2 | 2.2 | 2.2 | 2.2 | 64 | 64 | 64 | 64 | [37] |
| 2446-69-7 | 4-Hexylphenol | 31 | 2.0 | 23 | 29 | 100 | 45 | 100 | 100 | [29, 36] |
| 1137-42-4 | 4-Hydroxybenzophenone | 6.8 | 0 | 2.1 | 5.9 | 100 | 11 | 57 | 89 | [38, 39] |
| 36861-47-9 | 4-Methylbenzylidene Camphor | 14 | 0.88 | 2.1 | 11 | 11 | 7 | 11 | 11 | [40] |
| 1806-26-4 | 4-Octylphenol | 61 | 18 | 59 | 61 | 100 | 71 | 95 | 99 | [7, 8] |
| 98-54-4 | 4-Tert-Butylphenol | 691 | 0.17 | 32 | 495 | 100 | 18 | 94 | 100 | [29, 36, 41, 42] |
| 140-66-9 | 4-Tert-Octylphenol | 78 | 1.8 | 6.4 | 58 | 100 | 8 | 98 | 100 | [11, 17, 18, 25, 29, 36, 42–44] |
| 4184-79-6 | 5,6-Dimethyl-1H-benzotriazole | 6.3 | 0.01 | 2.9 | 6.1 | 100 | 48 | 79 | 98 | [2, 17, 45] |
| 94-97-3 | 5-Chloro-1H-benzotriazole | 32 | 0.02 | 6.5 | 26 | 100 | 45 | 91 | 100 | [2, 3, 45] |
| 136-85-6 | 5-Methyl-1H-Benzotriazole | 302 | 0.13 | 66 | 227 | 100 | 34 | 100 | 100 | [2, 45, 46] |
| 30560-19-1 | Acephate | 1670 | 59 | 310 | 1398 | 100 | 83 | 100 | 100 | [47, 48] |
| 33665-90-6 | Acesulfame | 1430 | 504 | 967 | 1337 | 100 | 100 | 100 | 100 | [2, 45] |
| 103-90-2 | Acetaminophen | 1132 | 0 | 11 | 125 | 100 | 0 | 97 | 100 | [2, 45, 49–62] |
| 135410-20-7 | Acetamiprid | 19 | 0.90 | 1.1 | 32 | 100 | 81 | 90 | 100 | [46, 63] |
| 34256-82-1 | Acetochlor | 18 | 0.40 | 2.0 | 14 | 100 | 8 | 48 | 86 | [44, 63–65] |
| 98-86-2 | Acetophenone | 0.16 | 0.16 | 0.16 | 0.16 | 0 | 0 | 0 | 0 | [52] |
| 127-74-2 | Acetylsulfadiazine | 176 | 128 | 152 | 171 | 100 | 100 | 100 | 100 | [37] |
| 35255-37-9 | Acetylsulfamethazine | 4.0 | 4.0 | 4.0 | 4.0 | 100 | 100 | 100 | 100 | [37] |
| 21312-10-7 | Acetylsulfamethoxazole | 319 | 319 | 319 | 319 | 100 | 100 | 100 | 100 | [38] |
| 15972-60-8 | Alachlor | 6.3 | 1.7 | 4.0 | 5.8 | 71 | 71 | 71 | 71 | [63, 65] |
| 28981-97-7 | Alprazolam | 2.5 | 2.5 | 2.5 | 2.5 | 100 | 100 | 100 | 100 | [5] |
| 768-94-5 | Amantadine | 351 | 351 | 351 | 351 | 100 | 100 | 100 | 100 | [46] |
| 58-15-1 | Aminophenazone | 0.8 | 0.80 | 0.80 | 0.80 | 100 | 0 | 79 | 100 | [51] |
| 50-48-6 | Amitriptyline | 0.8 | 0.03 | 0.20 | 0.68 | 100 | 38 | 41 | 88 | [5, 66] |
| 88150-42-9 | Amlodipine | 0.68 | 0.27 | 0.48 | 0.64 | 0 | 0 | 0 | 0 | [56, 57] |
| 26787-78-0 | Amoxicillin | 23 | 2.7 | 17 | 23 | 100 | 90 | 95 | 100 | [60, 67] |
| 300-62-9 | Amphetamine | 2.8 | 0.03 | 0.10 | 1.7 | 97 | 3 | 24 | 80 | [68, 69] |
| 69-53-4 | Ampicillin | 17 | 17 | 17 | 17 | 0 | — | — | — | [67] |
| 63-05-8 | Androstenedione | 0.30 | 0.17 | 0.24 | 0.29 | 98 | 13 | 56 | 90 | [24, 70] |
| 53-41-8 | Androsterone | 0.02 | 0.02 | 0.02 | 0.02 | 1 | 1 | 1 | 1 | [6] |
| 60-80-0 | Antipyrine | 13 | 0 | 1.9 | 8.6 | 100 | 100 | 100 | 100 | [46, 52, 55, 60, 71] |
| 22839-47-0 | Aspartame | 0.03 | 0.03 | 0.03 | 0.03 | 22 | 19 | 21 | 22 | [52] |
| 29122-68-7 | Atenolol | 61 | 1.7 | 12 | 12 | 100 | 100 | 100 | 100 | [51, 62, 71] |
| 134523-00-5 | Atorvastatin | 0.08 | 0.03 | 0.06 | 0.08 | 22 | 14 | 18 | 21 | [56, 57] |
| 2163-68-0 | Atrazine-2-hydroxy | 600 | 90 | 345 | 549 | 100 | 100 | 100 | 100 | [47] |
| 6190-65-4 | Atrazine-desethyl | 20 | 10 | 15 | 19 | 100 | 100 | 100 | 100 | [47] |
| 19988-24-0 | Atrazine-desethyl-2-hydroxy | 360 | 180 | 270 | 342 | 100 | 100 | 100 | 100 | [47] |
| 1007-28-9 | Atrazine-Desisopropyl | 10 | 1.0 | 5.5 | 9.1 | 100 | 25 | 63 | 93 | [47] |
| 7313-54-4 | Atrazine-Desisopropyl-2-hydroxy | 1080 | 220 | 650 | 994 | 100 | 100 | 100 | 100 | [47] |
| 83905-01-5 | Azithromycin | 86 | 0.03 | 4.4 | 25 | 100 | 0 | 80 | 100 | [50, 54, 57, 60, 62, 71–79] |
| 131860-33-8 | Azoxystrobin | 10 | 10 | 10 | 10 | 100 | 100 | 100 | 100 | [46] |
| 25057-89-0 | Bentazone | 50 | 20 | 35 | 47 | 100 | 100 | 100 | 100 | [47] |
| 119-61-9 | Benzophenone | 407 | 132 | 317 | 389 | 100 | 100 | 100 | 100 | [38, 39] |
| 131-56-6 | Benzophenone-1 | 7.5 | 1.2 | 3.4 | 6.7 | 100 | 14 | 80 | 93 | [38, 39, 51] |
| 131-55-5 | Benzophenone-2 | 1.3 | 0.28 | 0.82 | 1.2 | 52 | 0 | 17 | 41 | [38, 39] |
| 131-57-7 | Benzophenone-3 | 1180 | 0.03 | 1.5 | 176 | 100 | 0 | 70 | 100 | [38–40, 51, 52] |
| 4065-45-6 | Benzophenone-4 | 54 | 0.35 | 34 | 47 | 100 | 0 | 78 | 100 | [38, 39, 51] |
| 95-14-7 | Benzotriazole | 276 | 0.14 | 56 | 227 | 100 | 92 | 100 | 100 | [2, 3, 45, 46] |
| 519-09-5 | Benzoylecgonine | 0.02 | 0.02 | 0.02 | 0.02 | 22 | 22 | 22 | 22 | [69] |
| 85-68-7 | Benzyl butyl phthalate | 491 | 0.007 | 131 | 422 | 100 | 0 | 41 | 100 | [16, 80–83] |
| 94-18-8 | Benzyl paraben | 0.54 | 0.01 | 0.02 | 0.46 | 72 | 0 | 2 | 46 | [52, 84] |
| 41859-67-0 | Bezafibrate | 84 | 0.15 | 21 | 39 | 100 | 24 | 98 | 100 | [10, 51, 56, 57, 71, 85–90] |
| 82657-04-3 | Bifenthrin | 18 | 0.05 | 3.4 | 13 | 100 | 18 | 33 | 94 | [48, 63, 64, 91, 92] |
| 605-54-9 | Bis-(2-ethoxyethyl) phthalate | 13 | 0 | 0 | 12 | 31 | 0 | 0 | 22 | [80, 81, 93] |
| 259139-51-0 | Bis-(4-methyl-2-pentyl) phthalate | 1 | 0 | 0 | 0.90 | 25 | 0 | 0 | 15 | [80, 81, 93] |
| 117-83-9 | Bis-2-n-butoxyethyl phthalate | 135 | 0 | 1.0 | 113 | 80 | 0 | 3 | 78 | [80, 81, 93] |
| ‘80-05-7 | Bisphenol A | 572 | 0.02 | 30 | 163 | 100 | 0 | 100 | 100 | [7, 8, 10–12, 14, 16–19, 21, 23, 25–29, 35, 36, 40, 42, 43, 52, 94–103] |
| 1478-61-1 | Bisphenol AF | 2.2 | 0.21 | 0.97 | 1.8 | 100 | 100 | 100 | 100 | [98, 101] |
| 1571-75-1 | Bisphenol AP | 0.02 | 0.006 | 0.006 | 0.02 | 52 | 0 | 8 | 43 | [98] |
| 77-40-7 | Bisphenol B | 7.3 | 0.04 | 0.05 | 6.1 | 100 | 0 | 0 | 95 | [98, 99, 101] |
| 79-97-0 | Bisphenol C | 0.03 | 0.03 | 0.03 | 0.03 | 0 | 0 | 0 | 0 | [98] |
| 620-92-8 | Bisphenol F | 277 | 0.03 | 4.6 | 106 | 100 | 0 | 49 | 91 | [95, 98, 99, 101, 104] |
| 3236-71-3 | Bisphenol FL | 0.07 | 0.03 | 0.03 | 0.06 | 15 | 0 | 10 | 14 | [98] |
| ‘80-09-1 | Bisphenol S | 980 | 0.51 | 8.9 | 334 | 100 | 1 | 100 | 100 | [95, 98, 99, 101, 104, 105] |
| 843-55-0 | Bisphenol Z | 3.7 | 0.01 | 0.01 | 3.6 | 100 | 0 | 8 | 90 | [98, 99] |
| 1812-30-2 | Bromazepam | 1.1 | 1.1 | 1.1 | 1.1 | 100 | 100 | 100 | 100 | [5] |
| 18181-80-1 | Bromopropylate | 4.7 | 4.0 | 4.4 | 4.6 | 100 | 62 | 81 | 96 | [63] |
| 69327-76-0 | Buprofezin | 1.9 | 1.9 | 1.9 | 1.9 | 24 | 24 | 24 | 24 | [63] |
| 23184-66-9 | Butachlor | 14500 | 0.30 | 19 | 27 | 100 | 57 | 68 | 93 | [63, 64, 106] |
| 94-26-8 | Buthylparaben | 4.2 | 0 | 0.31 | 2.9 | 100 | 1 | 63 | 100 | [1, 2, 4, 84, 104, 107] |
| 58-08-2 | Caffeine | 4200 | 5.1 | 157 | 2214 | 100 | 58 | 100 | 100 | [2, 10, 19, 45, 49, 50, 52–55, 85, 86, 88–90, 107–110] |
| 6804-07-5 | Carbadox | 0.43 | 0.43 | 0.43 | 0.43 | 13 | 0 | 7 | 12 | [111] |
| 298-46-4 | Carbamazepine | 93 | 0.20 | 4.9 | 57 | 100 | 43 | 100 | 100 | [2, 5, 10, 13, 22, 45, 46, 49–56, 58, 59, 61, 62, 70, 71, 85–90, 107–109] |
| 10605-21-7 | Carbendazim | 334 | 8.8 | 15 | 128 | 100 | 86 | 100 | 100 | [1–4, 45, 46, 112] |
| 15686-71-2 | Cefalexin | 16 | 3.4 | 5.8 | 13 | 97 | 5 | 51 | 88 | [50, 67, 113, 114] |
| 63527-52-6 | Cefotaxime | 33 | 0.46 | 11 | 33 | 100 | 4 | 39 | 100 | [60, 67, 113] |
| 38821-53-3 | Cefradine | 5.7 | 5.7 | 5.7 | 5.7 | 5 | 5 | 5 | 5 | [113] |
| 13171-00-1 | Celestolide | 1.7 | 0 | 0.83 | 1.7 | 33 | 0 | 17 | 30 | [115, 116] |
| 56-75-7 | Chloramphenicol | 159 | 0.005 | 0.70 | 25 | 100 | 0 | 73 | 100 | [47, 50, 51, 58–61, 67, 70, 74, 78, 86, 88–90, 96, 117–135] |
| 57-62-5 | Chlorotetracycline | 6820 | 0 | 2.4 | 71 | 100 | 0 | 40 | 100 | [53, 54, 61, 75, 78, 79, 111, 113, 117–121, 123–127, 129, 130, 132–134, 136–144] |
| 2921-88-2 | Chlorpyrifos | 15 | 0 | 1.5 | 13 | 100 | 3 | 51 | 84 | [48, 65, 145, 146] |
| 51481-61-9 | Cimetidine | 4.1 | 0.31 | 1.4 | 2.9 | 100 | 30 | 100 | 100 | [50, 58, 59] |
| 85721-33-1 | Ciprofloxacin | 238 | 0.03 | 3.3 | 53 | 100 | 0 | 31 | 100 | [2, 37, 51, 60, 61, 72, 75, 77, 111, 114, 120, 123, 137, 138, 140–142, 147, 148] |
| 59729-33-8 | Citalopram | 0.6 | 0.30 | 0.45 | 0.57 | 24 | 0 | 12 | 21 | [66] |
| 81103-11-9 | Clarithromycin | 32 | 0.02 | 3.4 | 12 | 100 | 0 | 73 | 100 | [2, 45, 49, 50, 60, 62, 67, 70–75, 77, 78, 126, 132, 135, 138] |
| 37148-27-9 | Clenbuterol | 0.03 | 0.03 | 0.03 | 0.03 | 0 | 0 | 0 | 0 | [52] |
| 38083-17-9 | Climbazole | 70 | 0.01 | 8.4 | 43 | 100 | 32 | 100 | 100 | [1–3, 45, 112] |
| 18323-44-9 | Clindamycin | 9.9 | 9.9 | 9.9 | 9.9 | 100 | 100 | 100 | 100 | [49] |
| 882-09-7 | Clofibric acid | 87 | 0.12 | 2.7 | 34 | 100 | 0 | 81 | 100 | [2, 10, 52, 56, 57, 62, 90, 97, 149, 150] |
| 303-49-1 | Clomipramine | 0.5 | 0.08 | 0.29 | 0.46 | 24 | 0 | 12 | 21 | [66] |
| 23593-75-1 | Clotrimazole | 2.9 | 0.02 | 0.74 | 2.2 | 100 | 32 | 62 | 94 | [1–4, 45] |
| 50-36-2 | Cocaine | 0.02 | 0.02 | 0.02 | 0.02 | 19 | 19 | 19 | 19 | [69] |
| 76-57-3 | Codeine | 0.13 | 0.03 | 0.09 | 0.13 | 38 | 0 | 7 | 29 | [34, 52, 69] |
| 152-58-9 | Cortexolone | 0.007 | 0.007 | 0.007 | 0.007 | 7 | 7 | 7 | 7 | [24] |
| 50-22-6 | Corticosterone | 0.08 | 0.08 | 0.08 | 0.08 | 0 | 0 | 0 | 0 | [6] |
| 50-23-7 | Cortisol | 1.6 | 1.6 | 1.6 | 1.6 | 31 | 31 | 31 | 31 | [6] |
| ‘53-06-5 | Cortisone | 0.06 | 0.06 | 0.06 | 0.06 | 17 | 17 | 17 | 17 | [6] |
| 486-56-6 | Cotinine | 15 | 4.1 | 9.3 | 13 | 100 | 87 | 94 | 99 | [70, 107] |
| 26444-49-5 | Cresyl diphenyl phosphate | 0.45 | 0.03 | 0.45 | 0.41 | 100 | 49 | 100 | 95 | [30, 151] |
| 483-63-6 | Crotamiton | 0.4 | 0.03 | 0.03 | 0.33 | 87 | 14 | 14 | 72 | [52, 71] |
| 2636-26-2 | Cyanophos | 0.03 | 0.03 | 0.03 | 0.03 | 33 | 33 | 33 | 33 | [64] |
| 100-88-9 | Cyclamate | 102 | 66 | 84 | 98 | 96 | 88 | 92 | 95 | [2, 45] |
| 50-18-0 | Cyclophosphamide | 0.05 | 0.05 | 0.05 | 0.05 | 4 | 0 | 2 | 4 | [52] |
| 68359-37-5 | Cyfluthrin | 7.1 | 2.2 | 3.2 | 6.7 | 81 | 48 | 56 | 78 | [63, 64, 152] |
| 52315-07-8 | Cypermethrin | 40 | 1.3 | 3.3 | 28 | 100 | 33 | 67 | 94 | [48, 63, 65] |
| 121552-61-2 | Cyprodinil | 0.05 | 0.05 | 0.05 | 0.05 | 38 | 38 | 38 | 38 | [63] |
| 112398-08-0 | Danofloxacin | 2.2 | 0.05 | 0.21 | 1.2 | 7 | 0 | 4 | 7 | [37, 52, 111] |
| 134-62-3 | DEET | 237 | 2.6 | 30 | 164 | 100 | 93 | 100 | 100 | [1–4, 45, 55, 70, 71, 86, 88–90, 107–110, 112] |
| 53-43-0 | Dehydroepiandrosterone | 0.33 | 0.33 | 0.33 | 0.33 | 13 | 13 | 13 | 13 | [24] |
| 651-48-9 | Dehydroepiandrosterone sulfate | 0.35 | 0.35 | 0.35 | 0.35 | 98 | 98 | 98 | 98 | [24] |
| 64-85-7 | Desoxycorticosterone | 0.11 | 0.11 | 0.11 | 0.11 | 75 | 75 | 75 | 75 | [24] |
| 439-14-5 | Diazepam | 18 | 0.80 | 3.2 | 14 | 100 | 62 | 100 | 100 | [5, 46, 52, 58, 59] |
| 333-41-5 | Diazinon | 0.13 | 0.13 | 0.13 | 0.13 | 24 | 24 | 24 | 24 | [64, 153] |
| 15307-86-5 | Diclofenac | 810 | 0.08 | 10 | 69 | 100 | 10 | 100 | 100 | [2, 13, 22, 45, 50, 52, 56, 57, 60, 62, 85–90, 97, 108, 109, 149, 150] |
| 84-61-7 | Dicyclohexyl phthalate | 2.2 | 0 | 0.003 | 2.0 | 87 | 0 | 0 | 49 | [80–82, 93] |
| 87130-20-9 | Diethofencarb | 2.9 | 0.45 | 1.7 | 2.7 | 90 | 29 | 60 | 84 | [63] |
| 84-66-2 | Diethyl phthalate | 145 | 0 | 21 | 464 | 100 | 10 | 100 | 100 | [16, 80–83, 93, 154–159] |
| 56-53-1 | Diethylstilbestrol | 12 | 0 | 0.59 | 9.8 | 100 | 0 | 47 | 97 | [15, 17, 19, 25] |
| 119446-68-3 | Difenoconazole | 1.1 | 1.1 | 1.1 | 1.1 | 10 | 5 | 7 | 9 | [63] |
| 98106-17-3 | Difloxacin | 70 | 0.48 | 0.66 | 5.0 | 69 | 0 | 25 | 54 | [37, 111, 133, 137, 141] |
| 521-18-6 | Dihydrotestosterone | 0.17 | 0.17 | 0.17 | 0.17 | 1 | 1 | 1 | 1 | [24] |
| 84-69-5 | Di-iso-butyl phthalate | 18380 | 2.0 | 113 | 3270 | 100 | 55 | 100 | 100 | [80–83, 93, 154, 155, 158–160] |
| 26761-40-0 | Diisodecyl phthalate | 433 | 433 | 433 | 433 | 64 | 64 | 64 | 64 | [156] |
| 28553-12-0 | Diisononyl phthalate | 493 | 0 | 98 | 493 | 80 | 0 | 44 | 65 | [81, 82, 155, 156, 158] |
| 34933-06-7 | Diltiazem | 8.9 | 0.20 | 4.9 | 7.9 | 100 | 19 | 86 | 99 | [49, 53, 54, 71] |
| 131-11-3 | Dimethyl phthalate | 644 | 0.04 | 47 | 134 | 100 | 50 | 100 | 100 | [16, 80–83, 93, 154–159] |
| 117-82-8 | Dimethylglycol phthalate | 0 | 0 | — | 58 | 0 | — | — | 95 | [81, 82] |
| 551-92-8 | Dimetridazole | 9.4 | 9.4 | 9.4 | 9.4 | 84 | 84 | 84 | 84 | [50] |
| 84-75-3 | Di-n-hexyl phthalate | 16 | 0 | 0.003 | 13 | 60 | 0 | 33 | 55 | [80–82, 93] |
| 117-84-0 | Di-n-octyl phthalate | 62 | 0.007 | 19 | 60 | 100 | 7 | 18 | 100 | [80–82, 93, 156, 157, 159] |
| 84-76-4 | Dinonyl phthalate | 1 | 1.0 | 1.0 | 1.0 | 0 | 0 | 0 | 0 | [80] |
| 131-18-0 | Dipentyl phthalate | 0 | 0 | — | 31 | 0 | — | — | 84 | [80–82] |
| 58-73-1 | Diphenhydramine | 1.7 | 0.10 | 1.6 | 1.7 | 100 | 42 | 69 | 94 | [49, 51, 70] |
| 131-16-8 | Di-propyl phthalate | 0 | 0 | 0 | — | 75 | 0 | 10 | 62 | [93] |
| 3737-09-5 | Dipydidamole | 2.7 | 2.7 | 2.7 | 6.8 | 32 | 32 | 32 | 55 | [71] |
| 298-04-4 | Disulfoton | 480 | 0.49 | 6.4 | 385 | 100 | 38 | 78 | 96 | [64, 145, 146] |
| 330-54-1 | Diuron | 15 | 15 | 15 | 15 | 100 | 100 | 100 | 100 | [46] |
| 1668-19-5 | Doxepin | 0.6 | 0.60 | 0.60 | 0.60 | 100 | 100 | 100 | 100 | [5] |
| 564-25-0 | Doxycycline | 42 | 0 | 1.6 | 16 | 100 | 0 | 56 | 100 | [51, 53, 54, 61, 75, 76, 78, 79, 111, 114, 117, 118, 120, 121, 123, 124, 133, 137, 139, 143, 161] |
| 74011-58-8 | Eenoxacin | 98 | 0.81 | 4.9 | 51 | 100 | 3 | 33 | 96 | [74, 78, 132] |
| 93106-60-6 | Enrofloxacin | 243 | 0 | 1.7 | 24 | 100 | 0 | 37 | 99 | [52, 60, 61, 74, 76, 78, 111, 113, 119, 120, 123, 124, 126–128, 130–134, 137, 138, 140, 142–144, 147, 162, 163] |
| 299-42-3 | Ephedrine | 119 | 12 | 27 | 93 | 100 | 94 | 100 | 100 | [68] |
| 114-07-8 | Erythromycin | 891 | 0.005 | 11 | 119 | 100 | 3 | 100 | 100 | [2, 13, 22, 45, 47, 49–51, 53, 54, 56–61, 67, 71–79, 96, 111, 113, 114, 117, 120–124, 126–129, 131, 132, 134, 135, 138, 142, 147, 148, 162] |
| 29975-16-4 | Estazolam | 1 | 1.0 | 1.0 | 1.0 | 100 | 100 | 100 | 100 | [5] |
| 50-27-1 | Estriol | 62 | 0.02 | 3.2 | 27 | 95 | 0 | 6 | 100 | [6–8, 10, 11, 14, 20, 21, 23, 24, 57, 59, 67, 72, 73, 77, 79, 125, 128, 129, 131, 132, 134, 135, 147, 148] |
| 53-16-7 | Estrone | 472 | 0 | 5.6 | 32 | 100 | 0 | 9 | 100 | [6–12, 14–16, 18–21, 23–26, 28, 51, 100] |
| 938-73-8 | Ethenzamide | 0.02 | 0.02 | 0.02 | 0.02 | 0 | 0 | 0 | 0 | [52] |
| 563-12-2 | Ethion | 0.5 | 0.03 | 0.27 | 2.6 | 38 | 20 | 29 | 36 | [64, 106] |
| 17302-82-8 | Ethyl 3,5-dichloro-4-hydroxybenzoate | 27 | 9.4 | 11 | 24 | 100 | 100 | 100 | 100 | [84] |
| 120-47-8 | Ethyl paraben | 58 | 0.005 | 1.3 | 5.9 | 100 | 1 | 96 | 100 | [1–4, 45, 84, 104, 112] |
| 94-09-7 | Ethyl-4-aminobenzoate | 0.05 | 0.05 | 0.05 | 0.05 | 10 | 10 | 10 | 10 | [51] |
| 118-60-5 | Ethylhexyl salicylate | 3.9 | 3.9 | 3.9 | 3.9 | 100 | 100 | 100 | 100 | [51] |
| 5466-77-3 | Ethylhexyl methoxycinnamate | 7.75 | 7.75 | 7.75 | 7.75 | 100 | 100 | 100 | 100 | [51] |
| 53-42-9 | Etiocholanone | 0.003 | 0.003 | 0.003 | 0 | 6 | 6 | 6 | 6 | [24] |
| 80844-07-1 | Etofenprox | 1.7 | 1.7 | 1.7 | 1.7 | 5 | 5 | 5 | 5 | [63] |
| 76824-35-6 | Famotidine | 0.5 | 0.003 | 0.25 | 0.45 | 36 | 10 | 23 | 33 | [50, 107, 145] |
| 52-85-7 | Famphur | 0 | 0 | 0 | 8.7 | 27 | 27 | 27 | 27 | [145] |
| 122-14-5 | Fenitrothion | 0.18 | 0 | 0.10 | 1.3 | 19 | 0 | 19 | 19 | [48, 64, 146] |
| 3766-81-2 | Fenobucarb | 17 | 0.67 | 5.7 | 14 | 100 | 30 | 81 | 96 | [63, 106] |
| 49562-28-9 | Fenofibrate | 0.10 | 0.01 | 0.09 | 0.10 | 100 | 26 | 58 | 86 | [58, 59] |
| 29679-58-1 | Fenoprofen | 6.4 | 2.0 | 4.2 | 6.0 | 100 | 89 | 95 | 99 | [52] |
| 39515-41-8 | Fenpropathrin | 33 | 1.5 | 8.1 | 26 | 100 | 33 | 62 | 89 | [48, 63, 64, 92, 152] |
| 55-38-9 | Fenthion | 1.5 | 1.5 | 1.5 | 1.5 | 0 | 0 | 0 | 0 | [106] |
| 79660-72-3 | Fleroxacin | 12 | 0.31 | 3.5 | 8.9 | 100 | 9 | 18 | 100 | [37, 61, 78, 111, 141] |
| 73231-34-2 | Florfenicol | 141 | 0.005 | 7.4 | 71 | 100 | 8 | 99 | 100 | [47, 58–61, 72, 74, 96, 111, 113, 117–124] |
| 86386-73-4 | Fluconazole | 48 | 2.9 | 7.6 | 30 | 100 | 52 | 100 | 100 | [1–4, 45, 112] |
| 101463-69-8 | Flufenoxuron | 0.05 | 0.05 | 0.05 | 0.05 | 24 | 24 | 24 | 24 | [63] |
| 42835-25-6 | Flumequine | 56 | 0.10 | 1.6 | 30 | 100 | 5 | 84 | 100 | [37, 50, 60, 67, 121] |
| 54910-89-3 | Fluoxetine | 34 | 0.02 | 1.8 | 27 | 100 | 0 | 81 | 100 | [5, 51–54, 66, 108] |
| 85509-19-9 | Flusilazole | 1 | 1.0 | 1.0 | 1.0 | 33 | 10 | 21 | 31 | [63] |
| 1222-05-5 | Galaxolide | 204 | 14 | 18 | 167 | 100 | 67 | 96 | 99 | [97, 115, 116] |
| 112811-59-3 | Gatifloxacin | 51 | 17 | 34 | 47 | 100 | 100 | 100 | 100 | [37, 78] |
| 25812-30-0 | Gemfibrozil | 47 | 0.005 | 3.0 | 18 | 100 | 14 | 95 | 100 | [2, 45, 50–52, 56–59, 85–90, 97, 149] |
| 10238-21-8 | Glibenclamide | 0.5 | 0.50 | 0.50 | 0.50 | 0 | 0 | 0 | 0 | [71] |
| 126-07-8 | Griseofulvin | 7.3 | 7.3 | 7.3 | 7.3 | 32 | 32 | 32 | 32 | [52] |
| 141-83-3 | Guanylurea | 140 | 140 | 140 | 140 | 99 | 99 | 99 | 99 | [50] |
| 1085-12-7 | Heptyl Paraben | 0.01 | 0 | 0 | 0.01 | 8 | 0 | 1 | 7 | [84] |
| 90717-16-1 | Hydroxylimine | 1.7 | 0.10 | 0.10 | 1.2 | 82 | 0 | 14 | 66 | [68] |
| 15687-27-1 | Ibuprofen | 116 | 0.76 | 20 | 67 | 100 | 13 | 86 | 100 | [2, 13, 22, 45, 49–52, 56, 57, 60, 62, 97, 107–109, 149, 150] |
| 119515-38-7 | Icaridin | 102 | 0.06 | 0.06 | 82 | 92 | 69 | 69 | 88 | [2, 4, 45] |
| 138261-41-3 | Imidacloprid | 42 | 1.6 | 22 | 54 | 100 | 81 | 95 | 100 | [46, 63] |
| 53-86-1 | Indometacin | 74 | 0.16 | 1.8 | 52 | 100 | 47 | 100 | 100 | [10, 51, 52, 55, 58–60, 71, 86–90, 97] |
| 173584-44-6 | Indoxacarb | 0.4 | 0.40 | 0.40 | 0.40 | 67 | 67 | 67 | 67 | [63] |
| 66108-95-0 | Iohexol | 72 | 1.4 | 31 | 64 | 100 | 2 | 65 | 93 | [2, 45, 50] |
| 60166-93-0 | Iopamidol | 52 | 52 | 52 | 52 | 65 | 65 | 65 | 65 | [45] |
| 73334-07-3 | Iopromide | 12 | 12 | 12 | 12 | 69 | 69 | 69 | 69 | [45] |
| 26087-47-8 | Iprobenfos | 8.3 | 0.75 | 4.3 | 7.5 | 64 | 24 | 62 | 63 | [63, 145] |
| 514-53-4 | Isochlortetracycline | 3.5 | 0.01 | 1.8 | 3.2 | 91 | 91 | 91 | 91 | [37, 67] |
| 25311-71-1 | Isofenphos | 0 | 0 | 0 | 2.2 | 18 | 18 | 18 | 18 | [145] |
| 2631-40-5 | Isoprocarb | 7.4 | 1.6 | 4.5 | 6.8 | 95 | 62 | 79 | 92 | [63] |
| 50512-35-1 | Isoprothiolane | 876 | 0.55 | 0.65 | 0.64 | 52 | 14 | 33 | 49 | [63, 64, 164] |
| 34123-59-6 | Isoproturon | 148 | 0 | 37 | 133 | 100 | 56 | 100 | 100 | [46, 47, 110] |
| 16846-24-5 | Josamycin | 2.3 | 0.08 | 2.3 | 2.3 | 100 | 68 | 83 | 97 | [79, 121] |
| 6740-88-1 | Ketamine | 1.6 | 0.03 | 0.15 | 0.88 | 74 | 0 | 36 | 72 | [34, 68, 69] |
| 65277-42-1 | Ketoconazole | 0.87 | 0.02 | 0.35 | 0.81 | 70 | 19 | 31 | 61 | [2, 4, 45, 51] |
| 22071-15-4 | Ketoprofen | 129 | 0 | 68 | 116 | 100 | 33 | 100 | 100 | [10, 52, 86–89, 150] |
| 143390-89-0 | Kresoxim-methyl | 3.5 | 3.5 | 3.5 | 3.5 | 90 | 90 | 90 | 90 | [64] |
| 120-40-1 | Lauryl Diethanolamide | 48 | 48 | 48 | 48 | 100 | 100 | 100 | 100 | [46] |
| 100986-85-4 | Levofloxacin | 58 | 58 | 58 | 58 | 87 | 87 | 87 | 87 | [71] |
| 137-58-6 | Lidocaine | 0.26 | 0.26 | 0.26 | 0.26 | 100 | 100 | 100 | 100 | [107] |
| 154-21-2 | Lincomycin | 136 | 0.28 | 12 | 61 | 100 | 3 | 93 | 100 | [2, 45, 49, 53–55, 70–72, 111, 114, 122, 135, 138, 148] |
| 98079-51-7 | Lomefloxacin | 31 | 0 | 1.1 | 16 | 100 | 0 | 77 | 97 | [37, 76, 78, 111, 124, 126, 133, 137, 142] |
| 79794-75-5 | Loratadine | 0.03 | 0.03 | 0.03 | 0.03 | 5 | 0 | 3 | 5 | [52] |
| 846-49-1 | Lorazepam | 4.2 | 4.2 | 4.2 | 4.2 | 100 | 100 | 100 | 100 | [5] |
| 114798-26-4 | Losartan | 0.03 | 0.03 | 0.03 | 0.03 | 5 | 0 | 3 | 5 | [52] |
| 115550-35-1 | Marbofloxacin | 0.25 | 0.25 | 0.25 | 0.25 | 34 | 9 | 22 | 32 | [111] |
| 61-68-7 | Mefenamic acid | 25 | 0.09 | 3.4 | 16 | 100 | 47 | 79 | 100 | [10, 52, 60, 86–90, 97, 108, 149] |
| 3562-63-8 | Megestrol | 0.10 | 0.10 | 0.10 | 0.10 | 0 | 0 | 0 | 0 | [6] |
| 57837-19-1 | Metalaxyl | 14 | 7.9 | 11 | 13 | 100 | 71 | 71 | 94 | [46, 63] |
| 657-24-9 | Metformin | 460 | 460 | 460 | 460 | 99 | 99 | 99 | 99 | [50] |
| 914-00-1 | Methacycline | 1.2 | 1.2 | 1.2 | 1.2 | 87 | 87 | 87 | 87 | [138] |
| 76-99-3 | Methadone | 0.02 | 0.02 | 0.02 | 0.02 | 8 | 8 | 8 | 8 | [69] |
| 537-46-2 | Methamphetamine | 12 | 0.03 | 2.8 | 8.9 | 100 | 0 | 79 | 95 | [34, 68, 69] |
| 161050-58-4 | Methoxyfenozide | 0.5 | 0.50 | 0.50 | 0.50 | 90 | 90 | 90 | 90 | [63] |
| 3964-57-6 | Methyl 3-chloro-4-hydroxybenzoate | 0.47 | 0.35 | 0.41 | 0.46 | 82 | 11 | 46 | 75 | [84] |
| 99-76-3 | Methyl paraben | 387 | 0.16 | 9.6 | 23 | 100 | 1 | 100 | 100 | [1–4, 52, 84, 104, 112] |
| 51218-45-2 | Metolachlor | 33 | 0.40 | 20 | 45 | 100 | 43 | 94 | 100 | [48, 145, 146, 153, 165–170] |
| 1129-41-5 | Metolcarb | 0.17 | 0.17 | 0.17 | 0.17 | 0 | 0 | 0 | 0 | [106] |
| 51384-51-1 | Metoprolol | 452 | 0 | 55 | 246 | 100 | 45 | 100 | 100 | [50, 52, 56, 57, 70, 71, 85, 86, 88–90, 108] |
| 443-48-1 | Metronidazole | 9.1 | 9.1 | 9.1 | 9.1 | 86 | 86 | 86 | 86 | [50] |
| 24219-97-4 | Mianserin | 0.2 | 0.20 | 0.20 | 0.20 | 100 | 100 | 100 | 100 | [5] |
| 22916-47-8 | Miconazole | 4.1 | 0.005 | 0.30 | 2.4 | 100 | 32 | 82 | 100 | [1, 2, 4, 45, 52, 112] |
| 2212-67-1 | Molinate | 0.33 | 0.33 | 0.33 | 0.33 | 0 | 0 | 0 | 0 | [106] |
| 17090-79-8 | Monensin | 6.6 | 0.40 | 3.3 | 5.7 | 80 | 54 | 70 | 80 | [45, 72] |
| 4376-20-9 | Mono-(2-ethylhexyl) phthalate | 19 | 19 | 19 | 19 | 100 | 100 | 100 | 100 | [154] |
| 2306-33-4 | Monoethyl phthalate | 3.0 | 3.0 | 3.0 | 3.0 | 29 | 29 | 29 | 29 | [154] |
| 30833-53-5 | Mono-iso-butyl phthalate | 15 | 15 | 15 | 15 | 100 | 100 | 100 | 100 | [154] |
| 4376-18-5 | Monomethyl phthalate | 45 | 45 | 45 | 45 | 100 | 100 | 100 | 100 | [154] |
| 131-70-4 | Mono-n-butyl phthalate | 24 | 24 | 24 | 24 | 100 | 100 | 100 | 100 | [154] |
| 57-27-2 | Morphine | 0.02 | 0.02 | 0.02 | 0.02 | 0 | 0 | 0 | 0 | [69] |
| 151096-09-2 | Moxifloxacin | 11 | 0 | 0 | 11 | 22 | 7 | 14 | 21 | [37, 56, 57] |
| 81-14-1 | Musk ketone | 14 | 1.9 | 3.0 | 12 | 83 | 17 | 71 | 81 | [115, 116] |
| 81-15-2 | Musk xylene | 0.72 | 0 | 0.36 | 0.72 | 42 | 0 | 21 | 38 | [97, 115] |
| 88671-89-0 | Myclobutanil | 19 | 0.95 | 1.1 | 15 | 62 | 33 | 52 | 60 | [63, 64] |
| 83-15-8 | N-acetyl-4-amino Antipyrine | 63 | 63 | 63 | 63 | 100 | 100 | 100 | 100 | [46] |
| 101651-74-5 | N-acetylsulfapyridine | 40 | 40 | 40 | 40 | 100 | 100 | 100 | 100 | [37] |
| 389-08-2 | Nalidixic acid | 130 | 0 | 25 | 82 | 100 | 1 | 91 | 100 | [10, 37, 50, 70, 86, 88–90, 121] |
| 22204-53-1 | Naproxen | 29 | 0.03 | 1.0 | 28 | 100 | 0 | 74 | 100 | [46, 50–52, 71, 97, 108, 109, 149, 150] |
| 55134-13-9 | Narasin | 0.30 | 0.05 | 0.30 | 0.30 | 3 | 0 | 0 | 2 | [111, 138] |
| 4151-50-2 | N-ethyl perfluoroctane sulfonamide | 0.004 | 0.004 | 0.004 | 0 | 14 | 14 | 14 | 14 | [171] |
| 2991-50-6 | N-ethyl perfluorooctanesulfonamidoacetic acid | 0.02 | 0.02 | 0.02 | 0.02 | 57 | 57 | 57 | 57 | [172] |
| 66085-59-4 | Nimodipine | 0.06 | 0.01 | 0.06 | 0.06 | 47 | 0 | 11 | 45 | [58, 59] |
| 31506-32-8 | N-methyl perfluoroctane sulfonamide | 0.02 | 0.02 | 0.02 | 0.02 | 0 | 0 | 0 | 0 | [171] |
| 1088-11-5 | Nordiazepam | 4.2 | 0.40 | 2.3 | 3.8 | 100 | 2 | 51 | 90 | [5, 173] |
| 68-22-4 | Norethindrone | 0.53 | 0.53 | 0.53 | 0.53 | 78 | 78 | 78 | 78 | [42] |
| 70458-96-7 | Norfloxacin | 199 | 0 | 2.6 | 44 | 100 | 0 | 53 | 100 | [2, 19, 37, 45, 51, 56, 57, 60, 61, 67, 74–76, 78, 79, 111, 113, 114, 117, 119, 120, 123, 125–135, 137, 138, 140, 142, 143, 147, 148, 162, 163, 174, 175] |
| 6197-30-4 | Octocrylene | 49 | 0.30 | 1.3 | 35 | 100 | 60 | 81 | 100 | [46, 51, 52] |
| 1219-38-1 | Octyl paraben | 0.13 | 0.03 | 0.08 | 0.12 | 64 | 10 | 37 | 59 | [84] |
| 27193-28-8 | Octyl phenol | 54 | 0.20 | 1.6 | 34 | 100 | 0 | 75 | 100 | [27, 94, 103, 176] |
| 82419-36-1 | Ofloxacin | 1693 | 0 | 6.0 | 60 | 100 | 0 | 76 | 100 | [2, 19, 37, 45, 47, 51–54, 56, 57, 60, 61, 67, 74–79, 111, 114, 117, 119, 120, 122–135, 137, 138, 140–144, 147, 148, 161–163, 174, 175] |
| 7060-74-4 | Oleandomycin Phosphate | 0.07 | 0.07 | 0.07 | 0.07 | 60 | 60 | 60 | 60 | [79] |
| 73590-58-6 | Omeprazole | 0.03 | 0.005 | 0.005 | 0.02 | 73 | 0 | 32 | 61 | [58, 59] |
| 16773-42-5 | Ornidazole | 1.7 | 1.7 | 1.7 | 1.7 | 47 | 47 | 47 | 47 | [50] |
| 19666-30-9 | Oxadiazon | 135 | 135 | 135 | 135 | 0 | — | — | — | [164] |
| 604-75-1 | Oxazepam | 15 | 3.1 | 9.1 | 14 | 100 | 93 | 96 | 99 | [5, 173] |
| 28721-07-5 | Oxcarbazepine | 0.005 | 0.005 | 0.005 | 0.005 | 30 | 30 | 30 | 30 | [107] |
| 14698-29-4 | Oxolinic acid | 6.9 | 0.84 | 1.5 | 5.8 | 93 | 40 | 84 | 92 | [37, 121] |
| 79-57-2 | Oxytetracycline | 2134 | 0 | 6.6 | 75 | 100 | 0 | 62 | 100 | [2, 37, 45, 50, 51, 53, 54, 60, 61, 67, 75, 76, 78, 79, 111, 113, 114, 117–121, 123–130, 132–144, 147, 148, 174, 175] |
| 33069-62-4 | Paclitaxel | 0.26 | 0.07 | 0.26 | 0.26 | 0 | 0 | 0 | 0 | [58, 59] |
| 76738-62-0 | Paclobutrazol | 12 | 0.40 | 6.3 | 11 | 62 | 48 | 55 | 60 | [63] |
| 611-59-6 | Paraxanthine | 34 | 14 | 24 | 32 | 100 | 67 | 83 | 97 | [49, 70] |
| 61869-08-7 | Paroxetine | 0.36 | 0.18 | 0.27 | 0.34 | 12 | 0 | 6 | 11 | [66] |
| 70458-92-3 | Pefloxacin | 6.4 | 0.40 | 0.47 | 4.6 | 9 | 0 | 4 | 8 | [37, 111, 138] |
| 40487-42-1 | Pendimethalin | 0.09 | 0.09 | 0.09 | 0.09 | 43 | 43 | 43 | 43 | [64] |
| 61-33-6 | Penicillin G | 11 | 0.40 | 1.6 | 7.9 | 93 | 3 | 92 | 93 | [67, 74, 109, 114, 121] |
| 87-08-1 | Penicillin V | 35 | 0 | 18 | 35 | 92 | 0 | 46 | 83 | [109, 114] |
| 422-64-0 | Pentafluoropropionic acid | 0 | 0 | 0 | 0.83 | 3 | 3 | 3 | 3 | [177] |
| 6521-29-5 | Pentyl paraben | 0.02 | 0 | 0.008 | 0.02 | 0 | 0 | 0 | 0 | [84] |
| 375-73-5 | Perfluorobutane sulfonic acid | 3720 | 0.13 | 1.6 | 28 | 100 | 35 | 100 | 100 | [171, 172, 177–203] |
| 375-22-4 | Perfluorobutanoic | 4580 | 0 | 3.2 | 42 | 100 | 0 | 99 | 100 | [171, 177–183, 185–191, 193–196, 198–205] |
| 335-77-3 | Perfluorodecane sulfonate | 1.7 | 0.001 | 0.03 | 0.75 | 91 | 0 | 45 | 82 | [171, 182, 184–186, 188, 191, 195] |
| 335-76-2 | Perfluorodecanoate | 20 | 0.003 | 0.24 | 1.5 | 100 | 0 | 72 | 100 | [51, 171, 172, 177–183, 185–192, 194–197, 199, 202, 204–210] |
| 307-55-1 | Perfluorododecanoic acid | 3.3 | 0 | 0.06 | 0.55 | 100 | 0 | 24 | 100 | [51, 171, 172, 177, 180, 182, 183, 185, 186, 189, 192, 195, 197, 199, 204–209] |
| 375-92-8 | Perfluoroheptane sulfonate | 0.06 | 0.003 | 0.04 | 0.06 | 84 | 0 | 28 | 73 | [171, 184, 188] |
| 375-85-9 | Perfluoroheptanoic acid | 127 | 0 | 0.91 | 4.1 | 100 | 26 | 100 | 100 | [51, 171, 172, 177–183, 185–208, 210] |
| 67905-19-5 | Perfluorohexadecanoic acid | 13 | 0.002 | 0.04 | 0.05 | 46 | 0 | 18 | 42 | [171, 182, 185, 186, 192, 195] |
| 355-46-4 | Perfluorohexane sulfonate | 488 | 0.005 | 0.43 | 17 | 100 | 0 | 80 | 100 | [171, 172, 177–192, 194–198, 202, 205–208, 210–212] |
| 307-24-4 | Perfluorohexanoic acid | 120 | 0.005 | 1.6 | 20 | 100 | 19 | 100 | 100 | [51, 171, 172, 177–188, 190–199, 202–206, 212] |
| 375-95-1 | Perfluorononanoic acid | 14 | 0 | 0.52 | 2.1 | 102 | 9 | 100 | 100 | [51, 171, 172, 177–199, 201–210] |
| 16517-11-6 | Perfluorooctadecanoic acid | 0.24 | 0.01 | 0.04 | 0.04 | 38 | 0 | 11 | 19 | [182, 185, 192, 195] |
| 1763-23-1 | Perfluorooctane sulfonate | 357 | 0.02 | 1.8 | 14 | 100 | 13 | 98 | 100 | [51, 171, 172, 177–182, 184–192, 194–214] |
| 335-67-1 | Perfluorooctanoate | 372 | 0.18 | 9.6 | 65 | 100 | 54 | 100 | 100 | [171, 172, 177–194, 196–216] |
| 2706-90-3 | Perfluoropentanoic acid | 75 | 0 | 0.98 | 26 | 100 | 1 | 97 | 100 | [171, 177, 179–196, 198–200, 202–206, 210] |
| 376-06-7 | Perfluorotetradecanoic acid | 0.4 | 0 | 0.02 | 0.12 | 76 | 0 | 14 | 55 | [171, 177, 180, 182, 185, 186, 192, 194, 195, 208] |
| 72629-94-8 | Perfluorotridecanoate | 0.17 | 0.004 | 0.02 | 0.14 | 89 | 0 | 21 | 80 | [171, 182, 185, 186, 189, 192] |
| 2058-94-8 | Perfluoroundecanoate | 8.8 | 0 | 0.10 | 0.82 | 100 | 0 | 57 | 100 | [51, 171, 172, 177, 180, 182, 183, 185, 188, 189, 191, 192, 194–197, 199, 202, 204–206, 208–210] |
| 52645-53-1 | Permethrin | 9.0 | 1.5 | 5.0 | 9.3 | 71 | 10 | 62 | 70 | [63, 64, 92] |
| 15323-35-0 | Phantolide | 1.9 | 0 | 0.93 | 1.9 | 28 | 0 | 14 | 25 | [115, 116] |
| 99-96-7 | P-hydroxybenzoic acid | 181 | 0.05 | 175 | 180 | 100 | 86 | 95 | 99 | [84] |
| 51940-44-4 | Pipemidic acid | 7.2 | 7.2 | 7.2 | 7.2 | 53 | 53 | 53 | 53 | [37] |
| 28797-61-7 | Pirenzepine | 0.03 | 0.03 | 0.03 | 0.03 | 0 | 0 | 0 | 0 | [52] |
| 23103-98-2 | Pirimicarb | 0.5 | 0.50 | 0.50 | 0.50 | 50 | 50 | 50 | 50 | [106] |
| 19562-30-2 | Piromidic acid | 12 | 12 | 12 | 51 | 47 | 47 | 47 | 47 | [37] |
| 50-24-8 | Prednisolone | 0.09 | 0.09 | 0.09 | 0.09 | 0 | 0 | 0 | 0 | [6] |
| 27605-76-1 | Probenazole | 0.06 | 0.06 | 0.06 | 0.06 | 80 | 80 | 80 | 80 | [107] |
| 67747-09-5 | Prochloraz | 0.2 | 0.20 | 0.20 | 0.20 | 90 | 90 | 90 | 90 | [63] |
| 32809-16-8 | Procymidone | 0.95 | 0.95 | 0.95 | 0.95 | 10 | 10 | 10 | 10 | [63] |
| 57-83-0 | Progesterone | 0.02 | 0.003 | 0.009 | 0.01 | 1 | 0 | 1 | 1 | [6, 24] |
| 60207-90-1 | Propiconazole | 32 | 1.8 | 26 | 31 | 100 | 33 | 62 | 92 | [46, 63] |
| 525-66-6 | Propranolol | 11 | 0 | 1.2 | 2.7 | 100 | 0 | 33 | 92 | [13, 22, 50–52, 62, 86, 88–90, 108] |
| 94-13-3 | Propyl paraben | 192 | 0.02 | 1.9 | 6.8 | 100 | 1 | 100 | 100 | [1–4, 52, 84, 104] |
| 479-92-5 | Propyphenazone | 0.9 | 0.02 | 0.02 | 0.72 | 97 | 25 | 28 | 84 | [52, 71] |
| 53112-28-0 | Pyrimethanil | 0.4 | 0.40 | 0.40 | 0.40 | 33 | 33 | 33 | 33 | [63] |
| 13593-03-8 | Quinalphos | 12 | 7.2 | 9.8 | 12 | 100 | 100 | 100 | 95 | [48, 153] |
| 80214-83-1 | Roxithromycin | 402 | 0.005 | 2.8 | 26 | 100 | 0 | 100 | 100 | [2, 13, 22, 45, 47, 50, 51, 55, 57–61, 67, 70–75, 77–79, 96, 111, 117, 120–129, 131, 132, 134, 135, 142, 147, 148, 162] |
| 81-07-2 | Saccharin | 471 | 141 | 306 | 438 | 100 | 100 | 100 | 100 | [2, 45] |
| 69-72-7 | Salicylic acid | 478 | 19 | 35 | 390 | 100 | 100 | 100 | 100 | [2, 45, 149] |
| 53003-10-4 | Salinomycin | 7.0 | 1.6 | 4.7 | 6.6 | 100 | 100 | 100 | 100 | [2, 45, 135, 138, 148] |
| 98105-99-8 | Sarafloxacin | 4.9 | 0.43 | 0.66 | 3.6 | 100 | 0 | 28 | 83 | [61, 111, 130, 140] |
| 79617-96-2 | Sertraline | 1.7 | 0.17 | 0.33 | 1.4 | 100 | 0 | 24 | 85 | [51, 66] |
| 139755-83-2 | Sildenafil | 0.05 | 0.05 | 0.05 | 0.05 | 26 | 19 | 20 | 25 | [52, 107] |
| 122-34-9 | Simazine | 10 | 1.0 | 5.5 | 194 | 67 | 31 | 49 | 93 | [47] |
| 79902-63-9 | Simvastatin | 15 | 0.07 | 0.29 | 13 | 100 | 0 | 0 | 100 | [56–59] |
| 3930-20-9 | Sotalol | 0.25 | 0.25 | 0.25 | 0.25 | 6 | 0 | 3 | 5 | [52] |
| 110871-86-8 | Sparfloxacin | 67 | 67 | 67 | 67 | 100 | 100 | 100 | 100 | [78] |
| 1695-77-8 | Spectinomycin | 210 | 210 | 210 | 210 | 0 | — | — | — | [67] |
| 8025-81-8 | Spiramycin | 28 | 0.03 | 0.24 | 3.3 | 100 | 0 | 20 | 90 | [72, 79, 111, 121, 124, 135, 148] |
| 57-92-1 | Streptomycin | 152 | 152 | 152 | 152 | 0 | — | — | — | [67] |
| 56038-13-2 | Sucralose | 1140 | 265 | 980 | 1108 | 100 | 82 | 96 | 99 | [2, 45, 50] |
| 144-80-9 | Sulfacetamide | 15 | 0.02 | 1.4 | 9.6 | 100 | 2 | 30 | 100 | [73, 77, 111, 118, 121, 124, 134] |
| 80-32-0 | Sulfachloropyridazine | 121 | 0 | 0.70 | 28 | 100 | 0 | 66 | 97 | [37, 47, 61, 74, 76, 111, 113, 118, 121, 129, 132, 134, 135, 137, 139, 142, 148, 217–219] |
| 68-35-9 | Sulfadiazine | 287 | 0.004 | 3.8 | 52 | 100 | 0 | 87 | 100 | [45, 47, 51–54, 56–58, 60, 61, 72, 75, 77, 96, 111, 113, 117–119, 121, 122, 126, 130, 137, 138, 140–142, 147, 161, 162, 217–219] |
| 122-11-2 | Sulfadimethoxine | 166 | 0.11 | 1.2 | 9.9 | 87 | 0 | 36 | 61 | [71, 74, 118, 121, 134, 137, 138, 217, 218] |
| 155-91-9 | Sulfadimethoxypyrimidine | 2.6 | 0.03 | 0.72 | 2.0 | 93 | 24 | 53 | 84 | [119, 132] |
| 2447-57-6 | Sulfadoxine | 23 | 0.17 | 4.7 | 20 | 100 | 40 | 88 | 98 | [111, 118] |
| 57-67-0 | Sulfaguanidine | 74 | 1.2 | 3.2 | 3.8 | 100 | 50 | 92 | 99 | [37, 121, 217] |
| 127-79-7 | Sulfamerazine | 37 | 0 | 0.13 | 5.0 | 100 | 0 | 40 | 96 | [50–52, 58, 59, 61, 67, 71, 75, 76, 78, 79, 96, 118–121, 123, 127, 132, 134–136, 139, 141, 147, 148, 217] |
| 57-68-1 | Sulfamethazine | 264 | 0 | 3.5 | 37 | 100 | 2 | 86 | 100 | [37, 52, 78, 118, 121, 138, 141, 143, 217] |
| 144-82-1 | Sulfamethiazole | 1.2 | 0.02 | 0.46 | 0.98 | 67 | 0 | 32 | 62 | [2, 37, 45, 47, 49, 52–54, 56–61, 67, 73, 75, 76, 78, 79, 96, 111, 117–124, 126, 127, 129–131, 134–143, 148, 161, 162, 175, 217–220] |
| 723-46-6 | Sulfamethoxazole | 260 | 0.005 | 11 | 62 | 100 | 17 | 100 | 100 | [118, 147, 217] |
| 651-06-9 | Sulfameter | 25 | 0.009 | 1.2 | 13 | 100 | 0 | 27 | 98 | [2, 19, 37, 46, 47, 49–61, 67, 71–79, 96, 109, 111, 113, 117–123, 125–127, 130–132, 134–136, 138–144, 147, 148, 161, 162, 217–220] |
| 80-35-3 | Sulfamethoxypyridazine | 25 | 0.03 | 0.73 | 15 | 100 | 15 | 60 | 87 | [78, 118, 121, 134] |
| 1220-83-3 | Sulfamonomethoxine | 58 | 0 | 6.7 | 23 | 100 | 3 | 87 | 100 | [2, 37, 45, 70, 76, 111, 113, 114, 118, 121, 135, 138, 141, 148, 219, 220] |
| 63-74-1 | Sulfanilamide | 867 | 0.80 | 434 | 7.9 | 100 | 73 | 87 | 97 | [37, 217] |
| 144-83-2 | Sulfapyridine | 28 | 0 | 0.93 | 17 | 100 | 0 | 85 | 100 | [2, 37, 45, 47, 58, 59, 61, 67, 71, 78, 79, 96, 111, 120, 121, 123, 134, 135, 139, 148, 161, 219, 220] |
| 59-40-5 | Sulfaquinoxaline | 22 | 0.02 | 0.70 | 2.9 | 100 | 0 | 77 | 100 | [37, 58, 59, 79, 96, 111, 113, 114, 119, 120, 123, 132, 134, 135, 138, 148, 219] |
| 72-14-0 | Sulfathiazole | 46 | 0.008 | 0.16 | 8.6 | 100 | 0 | 42 | 100 | [37, 58, 59, 62, 67, 73, 75–79, 96, 111, 118, 120, 123, 127, 129, 132, 134, 217] |
| 127-69-5 | Sulfisoxazole | 80 | 0.01 | 4.8 | 59 | 100 | 7 | 40 | 100 | [76, 118, 124, 129, 217] |
| 15676-16-1 | Sulpiride | 355 | 1.9 | 114 | 170 | 100 | 75 | 100 | 100 | [10, 55, 71, 86, 88–90] |
| 10540-29-1 | Tamoxifen | 0.2 | 0.05 | 0.17 | 0.20 | 100 | 100 | 100 | 100 | [58, 59] |
| 107534-96-3 | Tebuconazole | 33 | 33 | 33 | 33 | 100 | 100 | 100 | 100 | [46] |
| 112410-23-8 | Tebufenozide | 0.1 | 0.10 | 0.10 | 0.10 | 48 | 48 | 48 | 48 | [63] |
| 846-50-4 | Temazepam | 5.5 | 1.1 | 3.3 | 5.1 | 100 | 98 | 99 | 100 | [5, 173] |
| 5915-41-3 | Terbuthylazine | 4.0 | 0.30 | 2.2 | 3.6 | 100 | 38 | 69 | 94 | [47] |
| 886-50-0 | Terbutryn | 61 | 61 | 61 | 61 | 100 | 100 | 100 | 100 | [46] |
| 58-22-0 | Testosterone | 0.03 | 0.007 | 0.02 | 0.03 | 6 | 2 | 4 | 6 | [6] |
| 79-94-7 | Tetrabromobisphenol A | 2.9 | 0.05 | 0.75 | 2.3 | 76 | 0 | 40 | 75 | [29, 36, 41, 101] |
| 79-95-8 | Tetrachlorobisphenol A | 3.0 | 3.0 | 3.0 | 3.0 | 80 | 80 | 80 | 80 | [101] |
| 60-54-8 | Tetracycline | 688 | 0 | 4.2 | 99 | 100 | 0 | 56 | 100 | [2, 37, 45, 51, 53, 54, 61, 67, 75, 76, 78, 79, 111, 113, 114, 117–121, 123–130, 132–144, 147, 148, 162, 174, 175] |
| 91465-08-6 | Lambda-cyhalothrin | 29 | 1.9 | 7.6 | 12 | 100 | 24 | 78 | 94 | [64, 92, 152] |
| 148-79-8 | Thiabendazole | 0.55 | 0.005 | 0.005 | 0.55 | 92 | 4 | 67 | 92 | [1, 4, 45, 52] |
| 153719-23-4 | Thiamethoxam | 9.3 | 9.3 | 9.3 | 28 | 100 | 100 | 100 | 100 | [46] |
| 15318-45-3 | Thiamphenicol | 217 | 0.07 | 3.3 | 71 | 100 | 0 | 100 | 100 | [47, 51, 58–61, 67, 74, 96, 120, 121, 123] |
| 59669-26-0 | Thiodicarb | 4.2 | 4.2 | 4.2 | 4.2 | 86 | 86 | 86 | 86 | [63] |
| 297-97-2 | Thionazin | 11 | 11 | 11 | 11 | 100 | 100 | 100 | 100 | [145] |
| 55297-95-5 | Tiamulin | 2.1 | 0.40 | 1.3 | 1.9 | 100 | 100 | 100 | 100 | [55, 71] |
| 108050-54-0 | Tilmicosin | 0.2 | 0.20 | 0.20 | 0.20 | 81 | 81 | 81 | 81 | [79] |
| 19387-91-8 | Tinidazole | 0.97 | 0.97 | 0.97 | 0.97 | 30 | 30 | 30 | 30 | [50] |
| 21145-77-7 | Tonalide | 25 | 2.0 | 4.8 | 21 | 83 | 44 | 71 | 81 | [97, 115, 116] |
| 68140-48-7 | Traseolide | 0 | 0 | — | — | 0 | 0 | 0 | 0 | [115] |
| 513-02-0 | Tri(isopropyl) phosphate | 2.1 | 2.1 | 2.1 | 2.1 | 100 | 100 | 100 | 100 | [31] |
| 43121-43-3 | Triadimefon | 1.4 | 1.4 | 1.4 | 1.4 | 38 | 19 | 29 | 36 | [63] |
| 55219-65-3 | Triadimenol | 0.55 | 0.55 | 0.55 | 0.55 | 43 | 14 | 29 | 40 | [63] |
| 24017-47-8 | Triazophos | 12 | 0 | 11 | 12 | 100 | 27 | 95 | 100 | [63, 145] |
| 101-20-2 | Triclocarban | 43 | 0.02 | 3.8 | 16 | 100 | 52 | 100 | 100 | [1–4, 45, 49, 50, 52, 97, 109, 112] |
| 3380-34-5 | Triclosan | 235 | 0.05 | 12 | 50 | 100 | 0 | 100 | 100 | [1–4, 40, 45, 52, 97, 104, 109, 112] |
| 1330-78-5 | Tricresyl phosphate | 2.1 | 0.10 | 0.64 | 1.5 | 100 | 12 | 97 | 100 | [30, 32, 33, 151, 221] |
| 78-40-0 | Triethyl phosphate | 807 | 0.30 | 55 | 683 | 100 | 88 | 100 | 100 | [30, 32, 33, 46, 145, 151, 221] |
| 1582-09-8 | Trifluralin | 4.5 | 0.05 | 0.16 | 3.6 | 14 | 10 | 12 | 14 | [64, 65, 91] |
| 126-71-6 | Tri-iso-butyl phosphate | 35 | 0.05 | 14 | 31 | 100 | 12 | 94 | 99 | [30, 31, 222] |
| 738-70-5 | Trimethoprim | 835 | 0 | 6.1 | 60 | 100 | 0 | 100 | 100 | [2, 37, 45, 47, 49–51, 53–57, 60–62, 67, 71–77, 85, 86, 88–90, 107, 109, 111, 127, 129, 130, 132, 135, 138–140, 142–144, 148, 217, 219, 220] |
| 512-56-1 | Trimethyl phosphate | 48 | 16 | 23 | 43 | 100 | 88 | 88 | 98 | [30, 32, 33] |
| 739-71-9 | Trimipramine | 0.30 | 0.05 | 0.17 | 0.27 | 24 | 0 | 12 | 21 | [66] |
| 126-73-8 | Tri-n-butyl phosphate | 79 | 0.008 | 5.7 | 79 | 100 | 0 | 100 | 100 | [30–33, 151, 221, 222] |
| 513-08-6 | Tri-n-propyl phosphate | 1.7 | 0.005 | 0.08 | 1.2 | 33 | 0 | 1 | 24 | [30, 151, 221] |
| 115-86-6 | Triphenyl phosphate | 2.8 | 0.15 | 1.1 | 2.6 | 100 | 50 | 86 | 100 | [30, 31, 33, 151, 221, 222] |
| 791-28-6 | Triphenylphosphine oxide | 5.5 | 5.5 | 5.5 | 5.5 | 100 | 100 | 100 | 100 | [222] |
| 13674-87-8 | Tris (1,3-dichloro-2-propyl) phosphate | 28 | 0.10 | 12 | 27 | 100 | 72 | 100 | 100 | [30–33, 151, 221, 222] |
| 78-51-3 | Tris (2-butoxyethyl) phosphate | 200 | 0.01 | 23 | 200 | 100 | 4 | 100 | 100 | [30–33, 151, 221] |
| 115-96-8 | Tris (2-chloroethyl) phosphate | 501 | 0.50 | 92 | 437 | 100 | 96 | 100 | 100 | [30–33, 46, 151, 221–223] |
| 13674-84-5 | Tris (2-chloroisopropyl) phosphate | 495 | 2.5 | 144 | 452 | 100 | 99 | 100 | 100 | [30–32, 151, 221–223] |
| 78-42-2 | Tris (2-ethylhexyl) phosphate | 2.0 | 0.01 | 0.11 | 1.5 | 100 | 0 | 51 | 90 | [30–33, 151] |
| 786-19-6 | Trithion | 1.2 | 1.2 | 1.2 | 1.2 | 20 | 20 | 20 | 20 | [106] |
| 1401-69-0 | Tylosin | 15 | 0 | 0.76 | 7.5 | 100 | 0 | 44 | 95 | [47, 53, 54, 61, 79, 114, 122, 124, 127, 142] |
| 137862-53-4 | Valsartan | 4.7 | 4.7 | 4.7 | 4.7 | 100 | 100 | 100 | 100 | [107] |
| 93413-69-5 | Venlafaxine | 2.0 | 0 | 0.36 | 1.5 | 51 | 14 | 42 | 51 | [50, 66, 70, 108] |
| 210880-92-5 | Clothianidin | 26 | 9.5 | 16 | 23 | 100 | 100 | 100 | 100 | [145, 146, 153] |
| 2310-17-0 | Phosalone | 28 | 28 | 28 | 28 | 100 | 100 | 100 | 100 | [48] |
| 41198-08-7 | Profenofos | 9.3 | 9.3 | 9.3 | 9.3 | 100 | 100 | 100 | 100 | [48] |
| 111988-49-9 | Thiacloprid | 0.11 | 0.03 | 0.08 | 0.40 | 0 | 0 | 51 | 86 | [224, 225] |

Fig. S1. Distribution of scores for each specific criterion with Equations S1–S13: (a) concentration; (b) frequency; (c) persistence; (d) bioaccumulation; (e) ecotoxicity; (f) human health effects

| UM1=C-CminCmax-Cmin | (Eq. S1) |
| --- | --- |
| UM2=log2(C)-log2(Cmin)log2(Cmax)-log2(Cmin ) | (Eq. S2) |
| UM3=log2(C)-log2(Cmin, 4)log2(Cmax)-log2(Cmin, 4) | (Eq. S3) |
| UE1=1-PNECeco-PNECeco,minPNECeco,max-PNECeco,min | (Eq. S4) |
| UE2=1-log10(PNECeco)-log10(PNECeco,min)log10(PNECeco,max)-log10(PNECeco,min) | (Eq. S5) |
| UE3=1-log10(PNECeco)-log10(PNECeco,min,4)log10(PNECeco,max,5)-log10(PNECeco,min,4) | (Eq. S6) |
| UH1=1-PNEChum-PNEChum,minPNEChum,max-PNEChum,min | (Eq. S7) |
| UH2=1-log10(PNEChum)-log10(PNEChum,min)log10(PNEChum,max)-log10(PNEChum,min) | (Eq. S8) |
| UH3=1-log10(PNEChum)-log10(PNEChum,min,5)log10(PNEChum,max)-log10(PNEChum,min,5) | (Eq. S9) |
| UB1=log10Dow-log10Dow,minlog10Dow,max-log10Dow,min | (Eq. S10) |
| UB2=log10Dow-log10Dow,minlog10Dow,max,5-log10Dow,min | (Eq. S11) |
| UP1=1-H-HminHmax-Hmin | (Eq. S12) |
| UP2=1-H-Hmin,31Hmax-Hmin,31 | (Eq. S13) |

Fig. S2. Box plot of EOC hazard potentials (HPs) for each chemical category.

Fig. S3. Box plot of EOC concentrations for each chemical category. Note that the sweetener category uses the secondary y-axis.

Fig. S4. Box plot of EOC detection frequencies for each chemical category.

Reference

[1] Liu WR, Zhao JL, Liu YS, Chen ZF, Yang YY, Zhang QQ, et al. Biocides in the Yangtze River of China: spatiotemporal distribution, mass load and risk assessment. Environ Pollut 2015; 200: 53-63.

[2] Yang YY, Liu WR, Liu YS, Zhao JL, Zhang QQ, Zhang M, et al. Suitability of pharmaceuticals and personal care products (PPCPs) and artificial sweeteners (ASs) as wastewater indicators in the Pearl River Delta, South China. Sci Total Environ 2017; 590-591: 611-619.

[3] Zhang NS, Liu YS, Van den Brink PJ, Price OR, Ying GG. Ecological risks of home and personal care products in the riverine environment of a rural region in South China without domestic wastewater treatment facilities. Ecotoxicol Environ Saf 2015; 122: 417-425.

[4] Liu WR, Yang YY, Liu YS, Zhao JL, Zhang QQ, Yao L, et al. Biocides in the river system of a highly urbanized region: A systematic investigation involving runoff input. Sci Total Environ 2018; 624: 1023-1030.

[5] Wu M, Xiang J, Que C, Chen F, Xu G. Occurrence and fate of psychiatric pharmaceuticals in the urban water system of Shanghai, China. Chemosphere 2015; 138: 486-493.

[6] Zhou LJ, Zhang BB, Zhao YG, Wu QL. Occurrence, spatiotemporal distribution, and ecological risks of steroids in a large shallow Chinese lake, Lake Taihu. Sci Total Environ 2016; 557-558: 68-79.

[7] Yang Y, Cao X, Zhang M, Wang J. Occurrence and distribution of endocrine-disrupting compounds in the Honghu Lake and East Dongting Lake along the Central Yangtze River, China. Environ Sci Pollut Res Int 2015; 22: 17644-17652.

[8] Wang W, Ndungu AW, Wang J. Monitoring of Endocrine-Disrupting Compounds in Surface Water and Sediments of the Three Gorges Reservoir Region, China. Arch Environ Contam Toxicol 2016; 71: 509-517.

[9] Wang J, Zhu Y. Occurrence and risk assessment of estrogenic compounds in the East Lake, China. Environ Toxicol Pharmacol 2017; 52: 69-76.

[10] Li Z, Xiang X, Li M, Ma Y, Wang J, Liu X. Occurrence and risk assessment of pharmaceuticals and personal care products and endocrine disrupting chemicals in reclaimed water and receiving groundwater in China. Ecotoxicol Environ Saf 2015; 119: 74-80.

[11] Wang S, Zhu Z, He J, Yue X, Pan J, Wang Z. Steroidal and phenolic endocrine disrupting chemicals (EDCs) in surface water of Bahe River, China: Distribution, bioaccumulation, risk assessment and estrogenic effect on Hemiculter leucisculus. Environ Pollut 2018; 243: 103-114.

[12] Shi J, Liu X, Chen Q, Zhang H. Spatial and seasonal distributions of estrogens and bisphenol A in the Yangtze River Estuary and the adjacent East China Sea. Chemosphere 2014; 111: 336-343.

[13] Liu J, Lu G, Xie Z, Zhang Z, Li S, Yan Z. Occurrence, bioaccumulation and risk assessment of lipophilic pharmaceutically active compounds in the downstream rivers of sewage treatment plants. Sci Total Environ 2015; 511: 54-62.

[14] Zhang A, Li Y, Chen L. Distribution and seasonal variation of estrogenic endocrine disrupting compounds, N-nitrosodimethylamine, and N-nitrosodimethylamine formation potential in the Huangpu River, China. Journal of Environmental Sciences 2014; 26: 1023-1033.

[15] Rao K, Lei B, Li N, Ma M, Wang Z. Determination of estrogens and estrogenic activities in water from three rivers in Tianjin, China. Journal of Environmental Sciences 2013; 25: 1164-1171.

[16] Niu S, Zhang C. Endocrine Disrupting Compounds from the Source Water of the Huai River (Huainan City), China. Arch Environ Contam Toxicol 2018; 74: 471-483.

[17] Dan L, Wu S, Xu H, Zhang Q, Zhang S, Shi L, et al. Distribution and bioaccumulation of endocrine disrupting chemicals in water, sediment and fishes in a shallow Chinese freshwater lake: Implications for ecological and human health risks. Ecotoxicol Environ Saf 2017; 140: 222-229.

[18] Yang J, Li H, Ran Y, Chan K. Distribution and bioconcentration of endocrine disrupting chemicals in surface water and fish bile of the Pearl River Delta, South China. Chemosphere 2014; 107: 439-446.

[19] Yan Z, Yang X, Lu G, Liu J, Xie Z, Wu D. Potential environmental implications of emerging organic contaminants in Taihu Lake, China: comparison of two ecotoxicological assessment approaches. Sci Total Environ 2014; 470-471: 171-179.

[20] Huang B, Wang B, Ren D, Jin W, Liu J, Peng J, et al. Occurrence, removal and bioaccumulation of steroid estrogens in Dianchi Lake catchment, China. Environ Int 2013; 59: 262-273.

[21] Wang Y, Wang Q, Hu L, Lu G, Li Y. Occurrence of estrogens in water, sediment and biota and their ecological risk in Northern Taihu Lake in China. Environ Geochem Health 2015; 37: 147-156.

[22] Xie Z, Lu G, Liu J, Yan Z, Ma B, Zhang Z, et al. Occurrence, bioaccumulation, and trophic magnification of pharmaceutically active compounds in Taihu Lake, China. Chemosphere 2015; 138: 140-147.

[23] Xu W, Yan W, Huang W, Miao L, Zhong L. Endocrine-disrupting chemicals in the Pearl River Delta and coastal environment: sources, transfer, and implications. Environ Geochem Health 2014; 36: 1095-1104.

[24] Yao B, Li R, Yan S, Chan SA, Song W. Occurrence and estrogenic activity of steroid hormones in Chinese streams: A nationwide study based on a combination of chemical and biological tools. Environ Int 2018; 118: 1-8.

[25] Gong J, Duan D, Yang Y, Ran Y, Chen D. Seasonal variation and partitioning of endocrine disrupting chemicals in waters and sediments of the Pearl River system, South China. Environ Pollut 2016; 219: 735-741.

[26] Fan Z, Hu J, An W, Yang M. Detection and occurrence of chlorinated byproducts of bisphenol a, nonylphenol, and estrogens in drinking water of china: comparison to the parent compounds. Environ Sci Technol 2013; 47: 10841-10850.

[27] Shi W, Hu G, Chen S, Wei S, Cai X, Chen B, et al. Occurrence of estrogenic activities in second-grade surface water and ground water in the Yangtze River Delta, China. Environ Pollut 2013; 181: 31-37.

[28] Shi BY, Wang ZY, Liu JJ, Chen QW, Sun QG, Hu LM. Pollution characteristics of three estrogens in drinking water sources in Jiangsu reach of the Yangtze River. Acta Scientiae Circumstantiae 2018; 38: 875-883.

[29] Liu D, Liu J, Guo M, Xu H, Zhang S, Shi L, et al. Occurrence, distribution, and risk assessment of alkylphenols, bisphenol A, and tetrabromobisphenol A in surface water, suspended particulate matter, and sediment in Taihu Lake and its tributaries. Mar Pollut Bull 2016; 112: 142-150.

[30] Shi Y, Gao L, Li W, Wang Y, Liu J, Cai Y. Occurrence, distribution and seasonal variation of organophosphate flame retardants and plasticizers in urban surface water in Beijing, China. Environ Pollut 2016; 209: 1-10.

[31] Wang X, Zhu L, Zhong W, Yang L. Partition and source identification of organophosphate esters in the water and sediment of Taihu Lake, China. J Hazard Mater 2018; 360: 43-50.

[32] Liu Y, Song N, Guo R, Xu H, Zhang Q, Han Z, et al. Occurrence and partitioning behavior of organophosphate esters in surface water and sediment of a shallow Chinese freshwater lake (Taihu Lake): Implication for eco-toxicity risk. Chemosphere 2018; 202: 255-263.

[33] Xing L, Zhang Q, Sun X, Zhu H, Zhang S, Xu H. Occurrence, distribution and risk assessment of organophosphate esters in surface water and sediment from a shallow freshwater Lake, China. Sci Total Environ 2018; 636: 632-640.

[34] Li K, Du P, Xu Z, Gao T, Li X. Occurrence of illicit drugs in surface waters in China. Environ Pollut 2016; 213: 395-402.

[35] Ben W, Zhu B, Yuan X, Zhang Y, Yang M, Qiang Z. Occurrence, removal and risk of organic micropollutants in wastewater treatment plants across China: Comparison of wastewater treatment processes. Water Res 2018; 130: 38-46.

[36] Liu YH, Zhang SH, Ji GX, Wu SM, Guo RX, Cheng J, et al. Occurrence, distribution and risk assessment of suspected endocrine-disrupting chemicals in surface water and suspended particulate matter of Yangtze River (Nanjing section). Ecotoxicol Environ Saf 2017; 135: 90-97.

[37] Zhang Q, Jia A, Wan Y, Liu H, Wang K, Peng H, et al. Occurrences of three classes of antibiotics in a natural river basin: association with antibiotic-resistant Escherichia coli. Environ Sci Technol 2014; 48: 14317-14325.

[38] Wu MH, Li J, Xu G, Ma LD, Li JJ, Li JS, et al. Pollution patterns and underlying relationships of benzophenone-type UV-filters in wastewater treatment plants and their receiving surface water. Ecotoxicol Environ Saf 2018; 152: 98-103.

[39] Wu MH, Xie DG, Xu G, Sun R, Xia XY, Liu WL, et al. Benzophenone-type UV filters in surface waters: An assessment of profiles and ecological risks in Shanghai, China. Ecotoxicol Environ Saf 2017; 141: 235-241.

[40] Wu C, Huang X, Lin J, Liu J. Occurrence and fate of selected endocrine-disrupting chemicals in water and sediment from an urban lake. Arch Environ Contam Toxicol 2015; 68: 225-236.

[41] Chen M H, Guo M, Liu D, et al. Occurrence and distribution of typical endocrine disruptors in surface water and sediments from Taihu Lake and its tributaries[J

[42] Xu G, Ma S, Tang L, Sun R, Xiang J, Xu B, et al. Occurrence, fate, and risk assessment of selected endocrine disrupting chemicals in wastewater treatment plants and receiving river of Shanghai, China. Environ Sci Pollut Res Int 2016; 23: 25442-25450.

[43] Diao P, Chen Q, Wang R, Sun D, Cai Z, Wu H, et al. Phenolic endocrine-disrupting compounds in the Pearl River Estuary: Occurrence, bioaccumulation and risk assessment. Sci Total Environ 2017; 584-585: 1100-1107.

[44] Li X, Shang X, Luo T, Du X, Wang Y, Xie Q, et al. Screening and health risk of organic micropollutants in rural groundwater of Liaodong Peninsula, China. Environ Pollut 2016; 218: 739-748.

[45] Yang YY, Zhao JL, Liu YS, Liu WR, Zhang QQ, Yao L, et al. Pharmaceuticals and personal care products (PPCPs) and artificial sweeteners (ASs) in surface and ground waters and their application as indication of wastewater contamination. Sci Total Environ 2018; 616-617: 816-823.

[46] Peng Y, Fang W, Krauss M, Brack W, Wang Z, Li F, et al. Screening hundreds of emerging organic pollutants (EOPs) in surface water from the Yangtze River Delta (YRD): Occurrence, distribution, ecological risk. Environ Pollut 2018; 241: 484-493.

[47] Sun S, Chen Y, Lin Y, An D. Occurrence, spatial distribution, and seasonal variation of emerging trace organic pollutants in source water for Shanghai, China. Sci Total Environ 2018; 639: 1-7.

[48] Chen Y, Yu K, Hassan M, Xu C, Zhang B, Gin KY, et al. Occurrence, distribution and risk assessment of pesticides in a river-reservoir system. Ecotoxicol Environ Saf 2018; 166: 320-327.

[49] Wu C, Huang X, Witter JD, Spongberg AL, Wang K, Wang D, et al. Occurrence of pharmaceuticals and personal care products and associated environmental risks in the central and lower Yangtze river, China. Ecotoxicol Environ Saf 2014; 106: 19-26.

[50] Yao B, Yan S, Lian L, Yang X, Wan C, Dong H, et al. Occurrence and indicators of pharmaceuticals in Chinese streams: A nationwide study. Environ Pollut 2018; 236: 889-898.

[51] Yan Z, Yang H, Dong H, Ma B, Sun H, Pan T, et al. Occurrence and ecological risk assessment of organic micropollutants in the lower reaches of the Yangtze River, China: A case study of water diversion. Environ Pollut 2018; 239: 223-232.

[52] Lv M, Sun Q, Hu A, Hou L, Li J, Cai X, et al. Pharmaceuticals and personal care products in a mesoscale subtropical watershed and their application as sewage markers. J Hazard Mater 2014; 280: 696-705.

[53] Zhang P, Zhou H, Li K, Zhao X, Liu Q, Li D, et al. Occurrence of pharmaceuticals and personal care products, and their associated environmental risks in a large shallow lake in north China. Environ Geochem Health 2018.

[54] Zhang P, Zhou H, Li K, Zhao X, Liu Q, Li D, et al. Occurrence of pharmaceuticals and personal care products, and their associated environmental risks in Guanting Reservoir and its upstream rivers in north China. RSC Advances 2018; 8: 4703-4712.

[55] Lin T, Yu S, Chen W. Occurrence, removal and risk assessment of pharmaceutical and personal care products (PPCPs) in an advanced drinking water treatment plant (ADWTP) around Taihu Lake in China. Chemosphere 2016; 152: 1-9.

[56] Yan Q, Zhang Y-X, Kang J, Gan X-M, Xu-Y P, Guo J-S, et al. A Preliminary Study on the Occurrence of Pharmaceutically Active Compounds in the River Basins and Their Removal in Two Conventional Drinking Water Treatment Plants in Chongqing, China. CLEAN - Soil, Air, Water 2015; 43: 794-803.

[57]  Yan Q, Zi CF, Zhang YX, Gan XM, Peng XY, Gao X, Guo JS, Pollution level and ecological risk assessment of typical pharmaceutically active compounds in the river basins of main districts of Chongqing. Res Environ Sci 2013; 26: 1178-1185.

[58] Zhao H, Cao Z, Liu X, Zhan Y, Zhang J, Xiao X, et al. Seasonal variation, flux estimation, and source analysis of dissolved emerging organic contaminants in the Yangtze Estuary, China. Mar Pollut Bull 2017; 125: 208-215.

[59] Zhao H, Zhou JL, Zhang J. Tidal impact on the dynamic behavior of dissolved pharmaceuticals in the Yangtze Estuary, China. Sci Total Environ 2015; 536: 946-954.

[60] Lin H, Chen L, Li H, Luo Z, Lu J, Yang Z. Pharmaceutically active compounds in the Xiangjiang River, China: Distribution pattern, source apportionment, and risk assessment. Sci Total Environ 2018; 636: 975-984.

[61] Hu XL, Bao YF, Hu JJ, Liu YY, Yin DQ. Occurrence of 25 pharmaceuticals in Taihu Lake and their removal from two urban drinking water treatment plants and a constructed wetland. Environ Sci Pollut Res Int 2017; 24: 14889-14902.

[62] Zhou H, Ying T, Wang X, Liu J. Occurrence and preliminarily environmental risk assessment of selected pharmaceuticals in the urban rivers, China. Sci Rep 2016; 6: 34928.

[63] Chen M. Pollution satus survey and risks assessment of 103 pesticides in Jiulong River Estuary and Xiamen Western Sea. Xiamen: Xiamen University; 2009, 1-117.

[64] Huang QT. Simultaneously determination method for 36 pesticides in aquatic environment and its application. Xiamen, Xiamen University; 2008, 1-91.

[65] Xue N, Xu X, Jin Z. Screening 31 endocrine-disrupting pesticides in water and surface sediment samples from Beijing Guanting reservoir. Chemosphere 2005; 61: 1594-1606.

[66] Wu M, Xiang J, Chen F, Fu C, Xu G. Occurrence and risk assessment of antidepressants in Huangpu River of Shanghai, China. Environ Sci Pollut Res Int 2017; 24: 20291-20299.

[67] Li S, Shi W, Li H, Xu N, Zhang R, Chen X, et al. Antibiotics in water and sediments of rivers and coastal area of Zhuhai City, Pearl River estuary, south China. Sci Total Environ 2018; 636: 1009-1019.

[68] Zhang Y, Zhang T, Guo C, Lv J, Hua Z, Hou S, et al. Drugs of abuse and their metabolites in the urban rivers of Beijing, China: Occurrence, distribution, and potential environmental risk. Sci Total Environ 2017; 579: 305-313.

[69] Wang DG, Zheng QD, Wang XP, Du J, Tian CG, Wang Z, et al. Illicit drugs and their metabolites in 36 rivers that drain into the Bohai Sea and north Yellow Sea, north China. Environ Sci Pollut Res Int 2016; 23: 16495-16503.

[70] Sun J, Luo Q, Wang D, Wang Z. Occurrences of pharmaceuticals in drinking water sources of major river watersheds, China. Ecotoxicol Environ Saf 2015; 117: 132-140.

[71] Wang Z, Zhang XH, Huang Y, Wang H. Comprehensive evaluation of pharmaceuticals and personal care products (PPCPs) in typical highly urbanized regions across China. Environ Pollut 2015; 204: 223-232.

[72] Wang W, Gu X, Zhou L, Chen H, Zeng Q, Mao Z. Antibiotics in Crab Ponds of Lake Guchenghu Basin, China: Occurrence, Temporal Variations, and Ecological Risks. Int J Environ Res Public Health 2018; 15.

[73] Zhang R, Tang J, Li J, Zheng Q, Liu D, Chen Y, et al. Antibiotics in the offshore waters of the Bohai Sea and the Yellow Sea in China: occurrence, distribution and ecological risks. Environ Pollut 2013; 174: 71-77.

[74] Du J, Zhao H, Liu S, Xie H, Wang Y, Chen J. Antibiotics in the coastal water of the South Yellow Sea in China: Occurrence, distribution and ecological risks. Sci Total Environ 2017; 595: 521-527.

[75] Tong L, Huang S, Wang Y, Liu H, Li M. Occurrence of antibiotics in the aquatic environment of Jianghan Plain, central China. Sci Total Environ 2014; 497-498: 180-187.

[76] Ma Y, Li M, Wu M, Li Z, Liu X. Occurrences and regional distributions of 20 antibiotics in water bodies during groundwater recharge. Sci Total Environ 2015; 518-519: 498-506.

[77]  Li J, Zhang RJ, Wang RM, Zhang H, Jiang DJ, Zuo T, et al. Distribution characteristics and ecological risk assessment of antibiotic pollution in Xiaoqing River watershed. J Agro-Environ Sci 2016; 35: 1384-1391.

[78] Yao L, Wang Y, Tong L, Li Y, Deng Y, Guo W, et al. Seasonal variation of antibiotics concentration in the aquatic environment: a case study at Jianghan Plain, central China. Sci Total Environ 2015; 527-528: 56-64.

[79] Xu Z, Li T, Bi J, Wang C. Spatiotemporal heterogeneity of antibiotic pollution and ecological risk assessment in Taihu Lake Basin, China. Sci Total Environ 2018; 643: 12-20.

[80] Liu Y, Chen Z, Shen J. Occurrence and removal characteristics of phthalate esters from typical water sources in northeast china. J Anal Methods Chem 2013; 2013: 419349.

[81] Zhang ZM, Zhang HH, Zhang J, Wang QW, Yang GP. Occurrence, distribution, and ecological risks of phthalate esters in the seawater and sediment of Changjiang River Estuary and its adjacent area. Sci Total Environ 2018; 619-620: 93-102.

[82] Zhang ZM, Zhang HH, Zou YW, Yang GP. Distribution and ecotoxicological state of phthalate esters in the sea-surface microlayer, seawater and sediment of the Bohai Sea and the Yellow Sea. Environ Pollut 2018; 240: 235-247.

[83] Shi W, Deng D, Wang Y, Hu G, Guo J, Zhang X, et al. Causes of endocrine disrupting potencies in surface water in East China. Chemosphere 2016; 144: 1435-1442.

[84] Li W, Gao L, Shi Y, Wang Y, Liu J, Cai Y. Spatial distribution, temporal variation and risks of parabens and their chlorinated derivatives in urban surface water in Beijing, China. Sci Total Environ 2016; 539: 262-270.

[85] Wang, D., Sui, Q., Lu, S.G., Zhao, W.T., Qiu, Z.F., Yu, G., 2014. Concentrations and distribution of selected pharmaceuticals and personal care products in Huangpu River. China Environ. Sci. 34, 1897-1904.

[86] Dai G, Wang B, Huang J, Dong R, Deng S, Yu G. Occurrence and source apportionment of pharmaceuticals and personal care products in the Beiyun River of Beijing, China. Chemosphere 2015; 119: 1033-1039.

[87] Li W, Gao L, Shi Y, Liu J, Cai Y. Occurrence, distribution and risks of antibiotics in urban surface water in Beijing, China. Environ Sci Process Impacts 2015; 17: 1611-1619.

[88] Dai G, Wang B, Fu C, Dong R, Huang J, Deng S, et al. Pharmaceuticals and personal care products (PPCPs) in urban and suburban rivers of Beijing, China: occurrence, source apportionment and potential ecological risk. Environ Sci Process Impacts 2016; 18: 445-455.

[89] Wang B, Dai G, Deng S, Huang J, Wang Y, Yu G. Linking the environmental loads to the fate of PPCPs in Beijing: Considering both the treated and untreated wastewater sources. Environ Pollut 2015; 202: 153-159.

[90] Yang L, He JT, Su SH, Cui YF, Huang DL, Wang GC. Occurrence, distribution, and attenuation of pharmaceuticals and personal care products in the riverside groundwater of the Beiyun River of Beijing, China. Environ Sci Pollut Res Int 2017; 24: 15838-15851.

[91] Zong DL, Chang AM, Zhang GM, Guan YT, Liang D, Deng WB. Pollution investigation of pesticide hormones in river water of Shenzhen. Environ Monit Manag Technol 2009; 21: 39-43.

[92] Zhao LN. Residue and risk assessement of 7 kinds of pyrethroids in water environment in the Pearl River Delta. Shanghai: Shanghai Ocean University; 2014, p. 1-69.

[93] Zheng X, Zhang BT, Teng Y. Distribution of phthalate acid esters in lakes of Beijing and its relationship with anthropogenic activities. Sci Total Environ 2014; 476-477: 107-113.

[94] Lv X, Xiao S, Zhang G, Jiang P, Tang F. Occurrence and removal of phenolic endocrine disrupting chemicals in the water treatment processes. Sci Rep 2016; 6: 22860.

[95] Yamazaki E, Yamashita N, Taniyasu S, Lam J, Lam PK, Moon HB, et al. Bisphenol A and other bisphenol analogues including BPS and BPF in surface water samples from Japan, China, Korea and India. Ecotoxicol Environ Saf 2015; 122: 565-572.

[96] Shi X, Zhou JL, Zhao H, Hou L, Yang Y. Application of passive sampling in assessing the occurrence and risk of antibiotics and endocrine disrupting chemicals in the Yangtze Estuary, China. Chemosphere 2014; 111: 344-351.

[97] Peng FJ, Pan CG, Zhang M, Zhang NS, Windfeld R, Salvito D, et al. Occurrence and ecological risk assessment of emerging organic chemicals in urban rivers: Guangzhou as a case study in China. Sci Total Environ 2017; 589: 46-55.

[98] Jin H, Zhu L. Occurrence and partitioning of bisphenol analogues in water and sediment from Liaohe River Basin and Taihu Lake, China. Water Res 2016; 103: 343-351.

[99] Yan Z, Liu Y, Yan K, Wu S, Han Z, Guo R, et al. Bisphenol analogues in surface water and sediment from the shallow Chinese freshwater lakes: Occurrence, distribution, source apportionment, and ecological and human health risk. Chemosphere 2017; 184: 318-328.

[100]  Gong J, Huang W, Yang J, Ran Y, Chen DY, Yang Y, et al. Occurrence of colloid-bound endocrine-disrupting chemicals in the Pearl River, China. China Environ Sci 2015; 35: 617-623.

[101] Yang Y, Lu L, Zhang J, Yang Y, Wu Y, Shao B. Simultaneous determination of seven bisphenols in environmental water and solid samples by liquid chromatography-electrospray tandem mass spectrometry. J Chromatogr A 2014; 1328: 26-34.

[102] Liu HH, Xu YJ, Deng XX, Gong XH, Song XK, Li F, et al. Survey of bisphenol A contamination in Laizhou Bay. Prog Fish Sci 2013; 34: 16-20.

[103] Zhang YZ, Meng W, Zhang Y. Occurrence and partitioning of phenolic endocrine-disrupting chemicals (EDCs) between surface water and suspended particulate matter in the north Tai Lake basin, eastern China. Bull Environ Contam Toxicol 2014; 92: 148-153.

[104] Huang C, Wu LH, Liu GQ, Shi L, Guo Y. Occurrence and Ecological Risk Assessment of Eight Endocrine-Disrupting Chemicals in Urban River Water and Sediments of South China. Arch Environ Contam Toxicol 2018; 75: 224-235.

[105] Wan Y, Xia W, Yang S, Pan X, He Z, Kannan K. Spatial distribution of bisphenol S in surface water and human serum from Yangtze River watershed, China: Implications for exposure through drinking water. Chemosphere 2018; 199: 595-602.

[106] Hu KD, Zhao FY, Wan YQ. Determination of organophosphorous and carbamates pesticide residues in water by GC-MS. J Nanchang University (Nat Sci) 2007; 31: 459-462.

[107] Liu N, Jin XW, Xue LD, Shi JZ, Xu YJ. Concentrations distribution and ecological risk assessment of pharmaceuticals and personal care products in Taihu Lake. China Environ Sci 2017; 37, 3515-3522.

[108] Ma R, Wang B, Lu S, Zhang Y, Yin L, Huang J, et al. Characterization of pharmaceutically active compounds in Dongting Lake, China: Occurrence, chiral profiling and environmental risk. Sci Total Environ 2016; 557-558: 268-275.

[109] Zhu S, Chen H, Li J. Sources, distribution and potential risks of pharmaceuticals and personal care products in Qingshan Lake basin, Eastern China. Ecotoxicol Environ Saf 2013; 96: 154-159.

[110]  Hu FF. Pollution characteristic analysis of emerging pesticide contaminants of in a high-risk tributry of upper reaches of Dongjing river basin. Gansu, Lanzhou Jiaotong Univerisiy; 2016, p. 1-66.

[111] Zhou LJ, Wu QL, Zhang BB, Zhao YG, Zhao BY. Occurrence, spatiotemporal distribution, mass balance and ecological risks of antibiotics in subtropical shallow Lake Taihu, China. Environ Sci Process Impacts 2016; 18: 500-513.

[112] Chen ZF, Ying GG, Liu YS, Zhang QQ, Zhao JL, Liu SS, et al. Triclosan as a surrogate for household biocides: an investigation into biocides in aquatic environments of a highly urbanized region. Water Res 2014; 58: 269-279.

[113] Song C, Zhang C, Fan L, Qiu L, Wu W, Meng S, et al. Occurrence of antibiotics and their impacts to primary productivity in fishponds around Tai Lake, China. Chemosphere 2016; 161: 127-135.

[114] Chen Y, Chen H, Zhang L, Jiang Y, Gin K, He Y. Occurrence, distribution, and risk assessment of antibiotics in a subtropical river-reservoir system. Water 2018; 10.

[115] Jiang S, Wang L, Zheng M, Lou Y, Shi L. Determination and environmental risk assessment of synthetic musks in the water and sediments of the Jiaozhou Bay wetland, China. Environ Sci Pollut Res Int 2018; 25: 4915-4923.

[116] Lu B, Feng Y, Gao P, Zhang Z, Lin N. Distribution and fate of synthetic musks in the Songhua River, Northeastern China: influence of environmental variables. Environ Sci Pollut Res Int 2015; 22: 9090-9099.

[117]  Hu GJ, Mu S, Zhao YG, Zhang PP, Chen SL. Characteristics of antibiotic content in drinking water in typical counties and districts of Nanjing. Enviro. Chem 2015; 34: 192-193.

[118] Na G, Fang X, Cai Y, Ge L, Zong H, Yuan X, et al. Occurrence, distribution, and bioaccumulation of antibiotics in coastal environment of Dalian, China. Mar Pollut Bull 2013; 69: 233-237.

[119] Li Y, Li Q, Zhou K, Sun XL, Zhao LR, Zhang YB. Occurrence and distribution of the environmental pollutant antibiotics in Gaoqiao mangrove area, China. Chemosphere 2016; 147: 25-35.

[120] Chen K, Zhou JL. Occurrence and behavior of antibiotics in water and sediments from the Huangpu River, Shanghai, China. Chemosphere 2014; 95: 604-612.

[121] Li N, Zhang X, Wu W, Zhao X. Occurrence, seasonal variation and risk assessment of antibiotics in the reservoirs in North China. Chemosphere 2014; 111: 327-335.

[122]  Feng L, Cheng YR, Feng L, Zhang S, Liu YQ. Distribution of typical antibiotics and ecological risk assessment in main waters of Three Gorges Reservoir area. Res Environ Sci 2017; 30: 1031-1040.

[123] Yan C, Yang Y, Zhou J, Liu M, Nie M, Shi H, et al. Antibiotics in the surface water of the Yangtze Estuary: occurrence, distribution and risk assessment. Environ Pollut 2013; 175: 22-29.

[124]  Lei XN. Distribution and pollution levels of antibiotics from typical lakes in Xinjiang. Xinjiang: Shihezi University; 2014: 1-55.

[125] Chen Y, Leung KS, Wong JW, Selvam A. Preliminary occurrence studies of antibiotic residues in Hong Kong and Pearl River Delta. Environ Monit Assess 2013; 185: 745-754.

[126] Yao L, Wang Y, Tong L, Deng Y, Li Y, Gan Y, et al. Occurrence and risk assessment of antibiotics in surface water and groundwater from different depths of aquifers: A case study at Jianghan Plain, central China. Ecotoxicol Environ Saf 2017; 135: 236-242.

[127] Jiang Y, Li M, Guo C, An D, Xu J, Zhang Y, et al. Distribution and ecological risk of antibiotics in a typical effluent-receiving river (Wangyang River) in north China. Chemosphere 2014; 112: 267-274.

[128] Zhao S, Liu X, Cheng D, Liu G, Liang B, Cui B, et al. Temporal-spatial variation and partitioning prediction of antibiotics in surface water and sediments from the intertidal zones of the Yellow River Delta, China. Sci Total Environ 2016; 569-570: 1350-1358.

[129] Xu J, Zhang Y, Zhou C, Guo C, Wang D, Du P, et al. Distribution, sources and composition of antibiotics in sediment, overlying water and pore water from Taihu Lake, China. Sci Total Environ 2014; 497-498: 267-273.

[130]  Liu XH. Pollution level, source and ecological risk of typical antibiotics in the Dongting Lake, China. Shangdong: Shandong Normal University; 2017, p. 1-93.

[131] Xu W, Yan W, Li X, Zou Y, Chen X, Huang W, et al. Antibiotics in riverine runoff of the Pearl River Delta and Pearl River Estuary, China: concentrations, mass loading and ecological risks. Environ Pollut 2013; 182: 402-407.

[132]  Yan XS. Distribution, source and risk evaluation of typical antibiotics in Xiaoqing river basin. Shandong, Shandong Normal Univeristy; 2018, p. 1-84.

[133] Tang J, Chen HY, Shi TZ, Li XD, Hua RM. Chen Y. Occurrence of quinolones and tetracyclines antibiotics in the aquatic environment of Chaohu Lake. J Anhui Agric University 2013; 40: 1043-1048.

[134] Bai Y, Meng W, Xu J, Zhang Y, Guo C. Occurrence, distribution and bioaccumulation of antibiotics in the Liao River Basin in China. Environ Sci Process Impacts 2014; 16: 586-593.

[135] Wei XD. Contamination of antibiotics in typical emission sources and a river in Guangzhou. Guangzhou: Guangzhou Institute of Geochemistry, Chinese Academy of Sciences; 2018, p. 1-86.

[136] Wang YH, Yang LL, Li ZC, Liu JS. Source and distribution of typical antibiotics in the upper Huangpu River, Shanghai. Environ Pollut Prevent 2017; 39: 301-306.

[137] Tang J, Shi T, Wu X, Cao H, Li X, Hua R, et al. The occurrence and distribution of antibiotics in Lake Chaohu, China: seasonal variation, potential source and risk assessment. Chemosphere 2015; 122: 154-161.

[138] Chen H, Liu S, Xu XR, Zhou GJ, Liu SS, Yue WZ, et al. Antibiotics in the coastal environment of the Hailing Bay region, South China Sea: Spatial distribution, source analysis and ecological risks. Mar Pollut Bull 2015; 95: 365-373.

[139] Jiang L, Hu X, Xu T, Zhang H, Sheng D, Yin D. Prevalence of antibiotic resistance genes and their relationship with antibiotics in the Huangpu River and the drinking water sources, Shanghai, China. Sci Total Environ 2013; 458-460: 267-272.

[140] Liu X, Lu S, Meng W, Wang W. Occurrence, source, and ecological risk of antibiotics in Dongting Lake, China. Environ Sci Pollut Res Int 2018; 25: 11063-11073.

[141] Wang Z, Du Y, Yang C, Liu X, Zhang J, Li E, et al. Occurrence and ecological hazard assessment of selected antibiotics in the surface waters in and around Lake Honghu, China. Sci Total Environ 2017; 609: 1423-1432.

[142] Chen H, Jing L, Teng Y, Wang J. Characterization of antibiotics in a large-scale river system of China: Occurrence pattern, spatiotemporal distribution and environmental risks. Sci Total Environ 2018; 618: 409-418.

[143] Yan M, Xu C, Huang Y, Nie H, Wang J. Tetracyclines, sulfonamides and quinolones and their corresponding resistance genes in the Three Gorges Reservoir, China. Sci Total Environ 2018; 631-632: 840-848.

[144] Xu J, Xu Y, Wang H, Guo C, Qiu H, He Y, et al. Occurrence of antibiotics and antibiotic resistance genes in a sewage treatment plant and its effluent-receiving river. Chemosphere 2015; 119: 1379-1385.

[145] Li YY, Hong HS, Wang XH, Hong LY, Ye CX. Estimation of the sources of organophosphorus pesticides in Xiamen sea area. Acta Scientiae Circumstantiae 2005; 25: 1071-1077.

[146]  Zhou M, Shi L, L QS, Yang X. The concentration and seasonal variation of organophosphorus pesticide residues in the Pearl River estuary. China Environ Sci 2013; 33: 312-318. .

[147] Dong D, Zhang L, Liu S, Guo Z, Hua X. Antibiotics in water and sediments from Liao River in Jilin Province, China: occurrence, distribution, and risk assessment. Environmental Earth Sciences 2016; 75.

[148] Zhou ZH, Zhao JJ, Wei XD, Liu MS. Co-occurrence and ecological risk of antibiotics in surface water of Guangzhou section of Pearl River. Ecol Environ Sci 2017; 26: 1034-1041.

[149] Wang L, Chen F, Yi H, Chen SL, Zhao XM, Guo QW, et al. Occurrence and risk assessment of pharmaceutical and personal care products in the lower reaches of Dongjiang River. J Henan Horm Univ (Nat Sci Edit) 2014; 42: 79-84.

[150] Wen ZH, Chen L, Meng XZ, Duan YP, Zhang ZS, Zeng EY. Occurrence and human health risk of wastewater-derived pharmaceuticals in a drinking water source for Shanghai, East China. Sci Total Environ 2014; 490: 987-993.

[151] Zhang Y, Zheng X, Wei L, Sun R, Guo H, Liu X, et al. The distribution and accumulation of phosphate flame retardants (PFRs) in water environment. Sci Total Environ 2018; 630: 164-170.

[152] Gong DC. Pyrethroids pesticides residues and their behavior in a multimedium environment of Liangtan river basin. Chongqing Univ; 2013, p. 1-93.

[153] Liao XP. Study of distribution, oxidation and reduction mechanisms of organophosphorus pesticides in typical area of Jianghan plain, Wuhan. China Univ Geosci; 2016, p. 1-138.

[154] Jiang J, Mu D, Ding M, Zhang S, Zhang H, Hu J. Simultaneous determination of primary and secondary phthalate monoesters in the Taihu Lake: Exploration of sources. Chemosphere 2018; 202: 17-24.

[155] Li R, Liang J, Gong Z, Zhang N, Duan H. Occurrence, spatial distribution, historical trend and ecological risk of phthalate esters in the Jiulong River, Southeast China. Sci Total Environ 2017; 580: 388-397.

[156] Wen Z, Huang X, Gao D, Liu G, Fang C, Shang Y, et al. Phthalate esters in surface water of Songhua River watershed associated with land use types, Northeast China. Environ Sci Pollut Res Int 2018; 25: 7688-7698.

[157] He W, Qin N, Kong X, Liu W, He Q, Ouyang H, et al. Spatio-temporal distributions and the ecological and health risks of phthalate esters (PAEs) in the surface water of a large, shallow Chinese lake. Sci Total Environ 2013; 461-462: 672-680.

[158] Li R, Liang J, Duan H, Gong Z. Spatial distribution and seasonal variation of phthalate esters in the Jiulong River estuary, Southeast China. Mar Pollut Bull 2017; 122: 38-46.

[159] Li B, Hu X, Liu R, Zeng P, Song Y. Occurrence and distribution of phthalic acid esters and phenols in Hun River Watersheds. Environmental Earth Sciences 2015; 73: 5095-5106.

[160] Zheng H, Yu Y, Fei J, Ding Z, Chen XD. Investigation of organic contaminants in main source water and finished water in Jiangsu. J Environ Health 2015; 32(3): 222-224.

[161] Deng W, Li N, Zheng H, Lin H. Occurrence and risk assessment of antibiotics in river water in Hong Kong. Ecotoxicol Environ Saf 2016; 125: 121-127.

[162] Liang X, Chen B, Nie X, Shi Z, Huang X, Li X. The distribution and partitioning of common antibiotics in water and sediment of the Pearl River Estuary, South China. Chemosphere 2013; 92: 1410-1416.

[163] Chen G, Liu X, Tartakevosky D, Li M. Risk assessment of three fluoroquinolone antibiotics in the groundwater recharge system. Ecotoxicol Environ Saf 2016; 133: 18-24.

[164] Ouyang W, Cai G, Tysklind M, Yang W, Hao F, Liu H. Temporal-spatial patterns of three types of pesticide loadings in a middle-high latitude agricultural watershed. Water Res 2017; 122: 377-386.

[165] Yang HH. Distribution characteristic and health risk assessment of organophosphorus pesticides in surface water of Hubei province. Hubei: Cent China Norm Univ; 2013, p. 1-70.

[166]  Bai HY, Han B, Zhang L, Yang DF, Wang XR, Sun PX. Composition and distribution characteristics of organophosphorus pesticides in the sea water of Sanggou bay. Rock Miner anal; 2012, 632-637.

[167] Gao J, Liu L, Liu X, Zhou H, Lu J, Huang S, et al. The occurrence and spatial distribution of organophosphorous pesticides in Chinese surface water. Bull Environ Contam Toxicol 2009; 82: 223-229.

[168] Na T, Fang Z, Zhanqi G, Ming Z, Cheng S. The status of pesticide residues in the drinking water sources in Meiliangwan Bay, Taihu Lake of China. Environ Monit Assess 2006; 123: 351-370.

[169] Chen H, Zhu J, Li Z, Chen A, Zhang Q. The occurrence and risk assessment of five organophosphorus pesticides in river water from Shangyu, China. Environ Monit Assess 2016; 188: 614.

[170] Bai HY, Han B, Sun PX, Zheng L, Cao L, Yang DF, Wang XR. Temporal and spatial distributions of organophosphorus pesticides in the seawater and sediment of Sanggou Bay. Mar Sci 2013; 37: 54-61.

[171] Cai M, Zhao Z, Yang H, Yin Z, Hong Q, Sturm R, et al. Spatial distribution of per- and polyfluoroalkyl compounds in coastal waters from the East to South China Sea. Environ Pollut 2012; 161: 162-169.

[172] Chen M, Wang Q, Shan G, Zhu L, Yang L, Liu M. Occurrence, partitioning and bioaccumulation of emerging and legacy per- and polyfluoroalkyl substances in Taihu Lake, China. Sci Total Environ 2018; 634: 251-259.

[173] Wang C, Hou L, Li J, Xu Z, Gao T, Yang J, et al. Occurrence of diazepam and its metabolites in wastewater and surface waters in Beijing. Environ Sci Pollut Res Int 2017; 24: 15379-15389.

[174] Cheng D, Liu X, Wang L, Gong W, Liu G, Fu W, et al. Seasonal variation and sediment-water exchange of antibiotics in a shallower large lake in North China. Sci Total Environ 2014; 476-477: 266-275.

[175] Cheng D, Liu X, Zhao S, Cui B, Bai J, Li Z. Influence of the natural colloids on the multi-phase distributions of antibiotics in the surface water from the largest lake in North China. Sci Total Environ 2017; 578: 649-659.

[176] Chen R, Yin P, Zhao L, Yu Q, Hong A, Duan S. Spatial-temporal distribution and potential ecological risk assessment of nonylphenol and octylphenol in riverine outlets of Pearl River Delta, China. J Environ Sci (China) 2014; 26: 2340-2347.

[177] Sun Z, Zhang C, Yan H, Han C, Chen L, Meng X, et al. Spatiotemporal distribution and potential sources of perfluoroalkyl acids in Huangpu River, Shanghai, China. Chemosphere 2017; 174: 127-135.

[178] Zhou Z, Shi Y, Li W, Xu L, Cai Y. Perfluorinated compounds in surface water and organisms from Baiyangdian Lake in North China: source profiles, bioaccumulation and potential risk. Bull Environ Contam Toxicol 2012; 89: 519-524.

[179] Sun R, Wu M, Tang L, Li J, Qian Z, Han T, et al. Perfluorinated compounds in surface waters of Shanghai, China: Source analysis and risk assessment. Ecotoxicol Environ Saf 2018; 149: 88-95.

[180] Su HC, Liu YS, Pan CG, Chen J, He LY, Ying GG. Persistence of antibiotic resistance genes and bacterial community changes in drinking water treatment system: From drinking water source to tap water. Sci Total Environ 2018; 616-617: 453-461.

[181] Lu GH, Gai N, Zhang P, Piao HT, Chen S, Wang XC, et al. Perfluoroalkyl acids in surface waters and tapwater in the Qiantang River watershed-Influences from paper, textile, and leather industries. Chemosphere 2017; 185: 610-617.

[182] Chen H, Han J, Zhang C, Cheng J, Sun R, Wang X, et al. Occurrence and seasonal variations of per- and polyfluoroalkyl substances (PFASs) including fluorinated alternatives in rivers, drain outlets and the receiving Bohai Sea of China. Environ Pollut 2017; 231: 1223-1231.

[183] Liu Z, Lu Y, Wang T, Wang P, Li Q, Johnson AC, et al. Risk assessment and source identification of perfluoroalkyl acids in surface and ground water: Spatial distribution around a mega-fluorochemical industrial park, China. Environ Int 2016; 91: 69-77.

[184] Lu Z, Song L, Zhao Z, Ma Y, Wang J, Yang H, et al. Occurrence and trends in concentrations of perfluoroalkyl substances (PFASs) in surface waters of eastern China. Chemosphere 2015; 119: 820-827.

[185] Chen H, Wang X, Zhang C, Sun R, Han J, Han G, et al. Occurrence and inputs of perfluoroalkyl substances (PFASs) from rivers and drain outlets to the Bohai Sea, China. Environ Pollut 2017; 221: 234-243.

[186] Shao M, Ding G, Zhang J, Wei L, Xue H, Zhang N, et al. Occurrence and distribution of perfluoroalkyl substances (PFASs) in surface water and bottom water of the Shuangtaizi Estuary, China. Environ Pollut 2016; 216: 675-681.

[187] Chai JF, Lei PH, Xia XY, Xu G, Wang DJ, Sun R, et al. Pollution patterns and characteristics of perfluorinated compounds in surface water adjacent potential industrial emission categories of Shanghai, China. Ecotoxicol Environ Saf 2017; 145: 659-664.

[188] Zhang Y, Lai S, Zhao Z, Liu F, Chen H, Zou S, et al. Spatial distribution of perfluoroalkyl acids in the Pearl River of southern China. Chemosphere 2013; 93: 1519-1525.

[189] Zhou Z, Liang Y, Shi Y, Xu L, Cai Y. Occurrence and transport of perfluoroalkyl acids (PFAAs), including short-chain PFAAs in Tangxun Lake, China. Environ Sci Technol 2013; 47: 9249-9257.

[190] Zhao Z, Tang J, Mi L, Tian C, Zhong G, Zhang G, et al. Perfluoroalkyl and polyfluoroalkyl substances in the lower atmosphere and surface waters of the Chinese Bohai Sea, Yellow Sea, and Yangtze River estuary. Sci Total Environ 2017; 599-600: 114-123.

[191] Pan CG, Ying GG, Zhao JL, Liu YS, Jiang YX, Zhang QQ. Spatiotemporal distribution and mass loadings of perfluoroalkyl substances in the Yangtze River of China. Sci Total Environ 2014; 493: 580-587.

[192] Liu B, Zhang H, Xie L, Li J, Wang X, Zhao L, et al. Spatial distribution and partition of perfluoroalkyl acids (PFAAs) in rivers of the Pearl River Delta, southern China. Sci Total Environ 2015; 524-525: 1-7.

[193] Zhang YZ, Wang B, Wang W, Li WC, Huang J, Deng SB, et al. Occurrence and source apportionment of Per- and poly-fluorinated compounds (PFCs) in North Canal Basin, Beijing. Sci Rep 2016; 6: 36683.

[194] Piao HT, Jiao XC, Gai N, Chen S, Lu GH, Yin XC, et al. Perfluoroalkyl substances in waters along the Grand Canal, China. Chemosphere 2017; 179: 387-394.

[195] Chen S, Jiao XC, Gai N, Li XJ, Wang XC, Lu GH, et al. Perfluorinated compounds in soil, surface water, and groundwater from rural areas in eastern China. Environ Pollut 2016; 211: 124-131.

[196] Wang P, Lu Y, Wang T, Fu Y, Zhu Z, Liu S, et al. Occurrence and transport of 17 perfluoroalkyl acids in 12 coastal rivers in south Bohai coastal region of China with concentrated fluoropolymer facilities. Environ Pollut 2014; 190: 115-122.

[197] Chen X, Zhu L, Pan X, Fang S, Zhang Y, Yang L. Isomeric specific partitioning behaviors of perfluoroalkyl substances in water dissolved phase, suspended particulate matters and sediments in Liao River Basin and Taihu Lake, China. Water Res 2015; 80: 235-244.

[198] Ding G, Xue H, Yao Z, Wang Y, Ge L, Zhang J, et al. Occurrence and distribution of perfluoroalkyl substances (PFASs) in the water dissolved phase and suspended particulate matter of the Dalian Bay, China. Chemosphere 2018; 200: 116-123.

[199] Zhao P, Xia X, Dong J, Xia N, Jiang X, Li Y, et al. Short- and long-chain perfluoroalkyl substances in the water, suspended particulate matter, and surface sediment of a turbid river. Sci Total Environ 2016; 568: 57-65.

[200] Wang P, Lu Y, Wang T, Zhu Z, Li Q, Meng J, et al. Coupled production and emission of short chain perfluoroalkyl acids from a fast developing fluorochemical industry: Evidence from yearly and seasonal monitoring in Daling River Basin, China. Environ Pollut 2016; 218: 1234-1244.

[201] Pan G, Zhou Q, Luan X, Fu QS. Distribution of perfluorinated compounds in Lake Taihu (China): impact to human health and water standards. Sci Total Environ 2014; 487: 778-784.

[202] Liu WX, He W, Qin N, Kong XZ, He QS, Yang B, et al. Temporal-spatial distributions and ecological risks of perfluoroalkyl acids (PFAAs) in the surface water from the fifth-largest freshwater lake in China (Lake Chaohu). Environ Pollut 2015; 200: 24-34.

[203]  Zhang M, Tang FL, Yu YY, Xu JF, Li H, Wu MH, et al. Residue Concentration and Distribution Characteristics of Perfluorinated Compounds in Surface Water from Qiantang River in Hangzhou Section. Environ Sci 2015; 36: 4471-4478.

[204] Zhang Y, Meng W, Guo C, Xu J, Yu T, Fan W, et al. Determination and partitioning behavior of perfluoroalkyl carboxylic acids and perfluorooctanesulfonate in water and sediment from Dianchi Lake, China. Chemosphere 2012; 88: 1292-1299.

[205] Guo C, Zhang Y, Zhao X, Du P, Liu S, Lv J, et al. Distribution, source characterization and inventory of perfluoroalkyl substances in Taihu Lake, China. Chemosphere 2015; 127: 201-207.

[206] Yu N, Shi W, Zhang B, Su G, Feng J, Zhang X, et al. Occurrence of perfluoroalkyl acids including perfluorooctane sulfonate isomers in Huai River Basin and Taihu Lake in Jiangsu Province, China. Environ Sci Technol 2013; 47: 710-717.

[207] Wang T, Khim JS, Chen C, Naile JE, Lu Y, Kannan K, et al. Perfluorinated compounds in surface waters from Northern China: comparison to level of industrialization. Environ Int 2012; 42: 37-46.

[208] Zhou Y, Tao Y, Li H, Zhou T, Jing T, Zhou Y, et al. Occurrence investigation of perfluorinated compounds in surface water from East Lake (Wuhan, China) upon rapid and selective magnetic solid-phase extraction. Sci Rep 2016; 6: 38633.

[209] Wang B, Cao M, Zhu H, Chen J, Wang L, Liu G, et al. Distribution of perfluorinated compounds in surface water from Hanjiang River in Wuhan, China. Chemosphere 2013; 93: 468-473.

[210] Cao Y, Cao X, Wang H, Wan Y, Wang S. Assessment on the distribution and partitioning of perfluorinated compounds in the water and sediment of Nansi Lake, China. Environ Monit Assess 2015; 187: 611.

[211] Ma X, Shan G, Chen M, Zhao J, Zhu L. Riverine inputs and source tracing of perfluoroalkyl substances (PFASs) in Taihu Lake, China. Sci Total Environ 2018; 612: 18-25.

[212] Wang SL, Zhang LP, Zhang X, Sun JS, Li WT, Guo FL, et al. Characteristics of spatial distribution of perfluoroalkyl acids ( PFAAs) and their precursors in the water samples from the Sihe River and the effluents of wastewater treatment plants. Acta Scientiae Circumstantiae 2018; 38: 1549-1557.

[213] Xu HY, Zhu JQ, Wang W, Lu Y, Xu XL. The Concentration Levels of Perfluorooctane Sulfonate (PFOS) and Perfluorooctanoic Acid (PFOA) in Qiantang River Waters of China. Advanced Materials Research 2014; 955-959: 2106-2109.

[214] Pan CG, Ying GG, Liu YS, Zhang QQ, Chen ZF, Peng FJ, et al. Contamination profiles of perfluoroalkyl substances in five typical rivers of the Pearl River Delta region, South China. Chemosphere 2014; 114: 16-25.

[215] Qi Y, Huo S, Hu S, Xi B, Su J, Tang Z. Identification, characterization, and human health risk assessment of perfluorinated compounds in groundwater from a suburb of Tianjin, China. Environmental Earth Sciences 2016; 75.

[216] Zhang JJ, Xue XY, Huang CS, Chen MB, Zhang LJ, Ren MZ, et al. Survey of perfluorooctanoic acid and perfluorooctane sulfonate in surface water from Tongsha Reservoir of Pearl River Delta，South China Environ Chem 2017; 36: 2600-2608.

[217] Jin L, Jiang L, Han Q, Xue JY, Ye H, Cao GM, et al. Distribution characteristics and health risk assessment of thirteen sulfonamides antibiotics in a drinking water source in East China. Environ Sci 2016; 37: 2515-2520.

[218] Tang J, Zhang FH, Wang CC, Shi TZ, Li XD, Hua RM. Investigation of sulfonamide antibiotics residue in the water of Chaohu Lake and its inlet rivers. J Saf Environ 2014; 14: 334-338.

[219] Wang JW, Wei H, Yang XY, Sun BC, Zhang JT. Occurrence and ecological risk of sulfonamide antibiotics in the surface water of the Weihe Xi'an section. Environ Chem 2017; 36: 2574-2583.

[220] Li J, Sun MT, Mao LS, Liu GH, Huang GW. Current situation of antibiotic pollution in water source water of Shenzhen City and assessment of ecological risk and human health risk. Occup Health 2018; 34: 2121-2126.

[221] Ding J, Shen X, Liu W, Covaci A, Yang F. Occurrence and risk assessment of organophosphate esters in drinking water from Eastern China. Sci Total Environ 2015; 538: 959-965.

[222] Zhong M, Tang J, Mi L, Li F, Wang R, Huang G, et al. Occurrence and spatial distribution of organophosphorus flame retardants and plasticizers in the Bohai and Yellow Seas, China. Mar Pollut Bull 2017; 121: 331-338.

[223] Wang X, He Y, Lin L, Zeng F, Luan T. Application of fully automatic hollow fiber liquid phase microextraction to assess the distribution of organophosphate esters in the Pearl River Estuaries. Sci Total Environ 2014; 470-471: 263-269.

[224] Xiong J, Wang Z, Ma X, Li H, You J. Occurrence and risk of neonicotinoid insecticides in surface water in a rapidly developing region: Application of polar organic chemical integrative samplers. Sci Total Environ 2019; 648: 1305-1312.

[225] Zhang C, Tian D, Yi X, Zhang T, Ruan J, Wu R, et al. Occurrence, distribution and seasonal variation of five neonicotinoid insecticides in surface water and sediment of the Pearl Rivers, South China. Chemosphere 2019; 217: 437-446.